// File: applications/Apps/debugCommands.c

/*

* Copyright (c) 2006-2026, RT-Thread Development Team

*

* SPDX-License-Identifier: Apache-2.0

*

* Change Logs:

* Date           Author       Notes

* 2026-09-18     ideapad15s   Debug MSH commands for sensor testing

*/

#include "heads.h"

#include "linesensor.h"

#include "MethaneSensorApp.h"

#include "sht30App.h"

#include "o2SensorApp.h"

#include "freeModbusApp.h"

#include "stressSensorApp.h"

#include "huaweiCloudApp.h"

#include "stm32h7xx_hal.h"

/**

* @brief 显示所有传感器数据汇总

*/

static void show_all_sensors(int argc, char **argv)

{

rt_kprintf("\n");

rt_kprintf("========================================\n");

rt_kprintf("  Pipe Gallery Node - Sensor Status\n");

rt_kprintf("========================================\n");

rt_kprintf("\n");

/* 1. 火焰传感器 (Five-channel ADC) */

rt_uint8_t fire_count = 0;

rt_kprintf("1. Flame Sensors (5-channel ADC):\n");

rt_kprintf("   A1 (ADC0): %5u    ", g_adc_ch0);

if (g_adc_ch0 > 60000) {

rt_kprintf("[FIRE DETECTED!]\n");

fire_count++;

} else {

rt_kprintf("[Normal]\n");

}

rt_kprintf("   A2 (ADC1): %5u    ", g_adc_ch1);

if (g_adc_ch1 > 60000) {

rt_kprintf("[FIRE DETECTED!]\n");

fire_count++;

} else {

rt_kprintf("[Normal]\n");

}

rt_kprintf("   A3 (ADC3): %5u    ", g_adc_ch3);

if (g_adc_ch3 > 60000) {

rt_kprintf("[FIRE DETECTED!]\n");

fire_count++;

} else {

rt_kprintf("[Normal]\n");

}

rt_kprintf("   A4 (ADC4): %5u    ", g_adc_ch4);

if (g_adc_ch4 > 60000) {

rt_kprintf("[FIRE DETECTED!]\n");

fire_count++;

} else {

rt_kprintf("[Normal]\n");

}

rt_kprintf("   A5 (ADC5): %5u    ", g_adc_ch5);

if (g_adc_ch5 > 60000) {

rt_kprintf("[FIRE DETECTED!]\n");

fire_count++;

} else {

rt_kprintf("[Normal]\n");

}

rt_kprintf("   >> Overall: ");

if (fire_count >= 4) {

rt_kprintf("**FIRE ALARM** (%d/5 triggered)\n", fire_count);

} else {

rt_kprintf("Normal (%d/5 triggered)\n", fire_count);

}

rt_kprintf("\n");

/* 2. 温湿度传感器 SHT30 */

rt_kprintf("2. SHT30 Temperature & Humidity:\n");

int temp_int = (int)g_temperature_c;

int temp_dec = (int)((g_temperature_c - temp_int) * 100);

if (temp_dec < 0) temp_dec = -temp_dec;

int humi_int = (int)g_humidity_rh;

int humi_dec = (int)((g_humidity_rh - humi_int) * 100);

if (humi_dec < 0) humi_dec = -humi_dec;

rt_kprintf("   Temperature: %d.%02d °C\n", temp_int, temp_dec);

rt_kprintf("   Humidity:    %d.%02d %%RH\n", humi_int, humi_dec);

rt_kprintf("\n");

/* 3. 甲烷传感器 */

rt_kprintf("3. Methane Gas Sensor:\n");

rt_kprintf("   Concentration: %u ppm", g_methane_ppm);

if (g_methane_ppm > 10000)

rt_kprintf("  [DANGER!]\n");

else if (g_methane_ppm > 5000)

rt_kprintf("  [WARNING!]\n");

else

rt_kprintf("  [Normal]\n");

rt_kprintf("   LEL%%:          %u %%\n", g_methane_lel);

rt_kprintf("\n");

/* 4. 氧气传感器 */

rt_kprintf("4. O2 Sensor:\n");

int o2_int = (int)g_o2_concentration;

int o2_dec = (int)((g_o2_concentration - o2_int) * 10);

if (o2_dec < 0) o2_dec = -o2_dec;

rt_kprintf("   O2: %d.%d %%", o2_int, o2_dec);

if (g_o2_concentration < 19.5f)

rt_kprintf("  [LOW OXYGEN!]\n");

else if (g_o2_concentration > 23.5f)

rt_kprintf("  [HIGH OXYGEN!]\n");

else

rt_kprintf("  [Normal]\n");

rt_kprintf("\n");

/* 5. 位移传感器 */

rt_kprintf("5. Displacement Sensor (Modbus):\n");

int disp_int = (int)Displacement;

int disp_dec = (int)((Displacement - disp_int) * 1000);

if (disp_dec < 0) disp_dec = -disp_dec;

rt_kprintf("   Displacement: %d.%03d mm\n", disp_int, disp_dec);

rt_kprintf("\n");

/* 6. 应力传感器 */

rt_kprintf("6. Stress Sensor RDF-TC25/RDD-DH (Modbus):\n");

if (g_stress_sensor_online && g_stress_value_valid)

{

int stress_int = (int)g_stress_value_n;

int stress_dec = (int)((g_stress_value_n - stress_int) * 100);

if (stress_dec < 0) stress_dec = -stress_dec;

rt_kprintf("   Stress: %d.%02d N  [Online]\n", stress_int, stress_dec);

}

else if (g_stress_sensor_online)

{

rt_kprintf("   Stress: invalid unit/status  [Online]\n");

}

else

{

rt_kprintf("   Stress: ---.-- N  [Offline]\n");

}

rt_kprintf("\n");

/* 7. 电表（三相电流） */

rt_kprintf("7. Ammeter (3-Phase Current):\n");

for (int i = 0; i < 3; i++)

{

int curr_int = (int)Current[i];

int curr_dec = (int)((Current[i] - curr_int) * 100);

if (curr_dec < 0) curr_dec = -curr_dec;

rt_kprintf("   Phase %d Current: %d.%02d A\n", i, curr_int, curr_dec);

}

rt_kprintf("\n");

/* 8. 电表（三相电压） */

rt_kprintf("8. Ammeter (3-Phase Voltage):\n");

for (int i = 0; i < 3; i++)

{

int volt_int = (int)Voltage[i];

int volt_dec = (int)((Voltage[i] - volt_int) * 100);

if (volt_dec < 0) volt_dec = -volt_dec;

rt_kprintf("   Phase %d Voltage: %d.%02d V\n", i, volt_int, volt_dec);

}

rt_kprintf("\n========================================\n");

rt_kprintf("  Use 'sensors' to refresh this display\n");

rt_kprintf("========================================\n\n");

}

MSH_CMD_EXPORT_ALIAS(show_all_sensors, sensors, Show all sensor data);

/**

* @brief 显示火焰传感器详细信息

*/

static void show_flame(int argc, char **argv)

{

rt_uint8_t fire_count = 0;

rt_kprintf("\n========================================\n");

rt_kprintf("  Five-Channel Flame Sensor Status\n");

rt_kprintf("========================================\n");

rt_kprintf("  Channel  |  ADC Value  |  Status\n");

rt_kprintf("-----------|-------------|-------------\n");

rt_kprintf("  A1 (CH0) |    %5u    | ", g_adc_ch0);

if (g_adc_ch0 > 60000) {

rt_kprintf("FIRE!\n");

fire_count++;

} else {

rt_kprintf("Normal\n");

}

rt_kprintf("  A2 (CH1) |    %5u    | ", g_adc_ch1);

if (g_adc_ch1 > 60000) {

rt_kprintf("FIRE!\n");

fire_count++;

} else {

rt_kprintf("Normal\n");

}

rt_kprintf("  A3 (CH3) |    %5u    | ", g_adc_ch3);

if (g_adc_ch3 > 60000) {

rt_kprintf("FIRE!\n");

fire_count++;

} else {

rt_kprintf("Normal\n");

}

rt_kprintf("  A4 (CH4) |    %5u    | ", g_adc_ch4);

if (g_adc_ch4 > 60000) {

rt_kprintf("FIRE!\n");

fire_count++;

} else {

rt_kprintf("Normal\n");

}

rt_kprintf("  A5 (CH5) |    %5u    | ", g_adc_ch5);

if (g_adc_ch5 > 60000) {

rt_kprintf("FIRE!\n");

fire_count++;

} else {

rt_kprintf("Normal\n");

}

rt_kprintf("========================================\n");

rt_kprintf("  Fire Channels: %d/5\n", fire_count);

rt_kprintf("  Overall Status: ");

if (fire_count >= 4) {

rt_kprintf("**FIRE ALARM**\n");

} else {

rt_kprintf("Normal\n");

}

rt_kprintf("========================================\n");

rt_kprintf("  ADC Range: 0-65535 (16-bit)\n");

rt_kprintf("  Threshold: >60000 = Fire Detected\n");

rt_kprintf("  Alarm Rule: >= 4 channels triggered\n");

rt_kprintf("========================================\n\n");

}

MSH_CMD_EXPORT_ALIAS(show_flame, flame, Show flame sensor status);

/**

* @brief 显示环境传感器（温湿度+氧气+甲烷）

*/

static void show_environment(int argc, char **argv)

{

rt_kprintf("\n========================================\n");

rt_kprintf("  Environment Sensors\n");

rt_kprintf("========================================\n");

/* 温度 */

int temp_int = (int)g_temperature_c;

int temp_dec = (int)((g_temperature_c - temp_int) * 100);

if (temp_dec < 0) temp_dec = -temp_dec;

rt_kprintf("  Temperature: %d.%02d °C\n", temp_int, temp_dec);

/* 湿度 */

int humi_int = (int)g_humidity_rh;

int humi_dec = (int)((g_humidity_rh - humi_int) * 100);

if (humi_dec < 0) humi_dec = -humi_dec;

rt_kprintf("  Humidity:    %d.%02d %%RH\n", humi_int, humi_dec);

/* 氧气 */

int o2_int = (int)g_o2_concentration;

int o2_dec = (int)((g_o2_concentration - o2_int) * 10);

if (o2_dec < 0) o2_dec = -o2_dec;

rt_kprintf("  O2:          %d.%d %%", o2_int, o2_dec);

if (g_o2_concentration < 19.5f)

rt_kprintf("  [LOW!]\n");

else if (g_o2_concentration > 23.5f)

rt_kprintf("  [HIGH!]\n");

else

rt_kprintf("\n");

/* 甲烷 */

rt_kprintf("  Methane:     %u ppm", g_methane_ppm);

if (g_methane_ppm > 10000)

rt_kprintf("  [DANGER!]\n");

else if (g_methane_ppm > 5000)

rt_kprintf("  [WARNING!]\n");

else

rt_kprintf("\n");

rt_kprintf("  Methane LEL: %u %%\n", g_methane_lel);

rt_kprintf("========================================\n\n");

}

MSH_CMD_EXPORT_ALIAS(show_environment, env, Show environment sensors);

/**

* @brief 显示Modbus设备状态

*/

static void show_modbus(int argc, char **argv)

{

rt_kprintf("\n========================================\n");

rt_kprintf("  Modbus RTU Devices (UART3)\n");

rt_kprintf("========================================\n");

rt_kprintf("  Baudrate: 9600\n");

rt_kprintf("  Parity:   None\n");

rt_kprintf("========================================\n\n");

/* 位移传感器 */

rt_kprintf("1. Displacement Sensor:\n");

rt_kprintf("   Slave Address: 1\n");

int disp_int = (int)Displacement;

int disp_dec = (int)((Displacement - disp_int) * 1000);

if (disp_dec < 0) disp_dec = -disp_dec;

rt_kprintf("   Value: %d.%03d mm\n\n", disp_int, disp_dec);

/* 应力传感器 */

rt_kprintf("2. Stress Sensor RDF-TC25/RDD-DH:\n");

rt_kprintf("   Slave Address: 3\n");

if (g_stress_sensor_online && g_stress_value_valid)

{

int stress_int = (int)g_stress_value_n;

int stress_dec = (int)((g_stress_value_n - stress_int) * 100);

if (stress_dec < 0) stress_dec = -stress_dec;

rt_kprintf("   Value: %d.%02d N\n", stress_int, stress_dec);

rt_kprintf("   Status: Online\n\n");

}

else if (g_stress_sensor_online)

{

rt_kprintf("   Value: invalid N conversion\n");

rt_kprintf("   Raw: %d, Unit: %u, Status: 0x%04X\n", g_stress_raw_value,

g_stress_unit, g_stress_status);

rt_kprintf("   Status: Online\n\n");

}

else

{

rt_kprintf("   Value: ---.-- N\n");

rt_kprintf("   Status: Offline\n\n");

}

/* 电表 */

rt_kprintf("3. Ammeter (3-Phase):\n");

rt_kprintf("   Slave Address: 73\n");

rt_kprintf("   Voltage:\n");

for (int i = 0; i < 3; i++)

{

int volt_int = (int)Voltage[i];

int volt_dec = (int)((Voltage[i] - volt_int) * 100);

if (volt_dec < 0) volt_dec = -volt_dec;

rt_kprintf("     Phase %d: %d.%02d V\n", i, volt_int, volt_dec);

}

rt_kprintf("   Current:\n");

for (int i = 0; i < 3; i++)

{

int curr_int = (int)Current[i];

int curr_dec = (int)((Current[i] - curr_int) * 100);

if (curr_dec < 0) curr_dec = -curr_dec;

rt_kprintf("     Phase %d: %d.%02d A\n", i, curr_int, curr_dec);

}

rt_kprintf("\n========================================\n");

rt_kprintf("========================================\n\n");

}

MSH_CMD_EXPORT_ALIAS(show_modbus, modbus, Show Modbus device status);

/**

* @brief 持续监控传感器数据（按Ctrl+C退出）

*/

static void monitor_sensors(int argc, char **argv)

{

int interval = 1000; /* 默认1秒 */

if (argc == 2)

{

interval = atoi(argv[1]) * 1000;

if (interval < 100) interval = 100;

if (interval > 10000) interval = 10000;

}

rt_kprintf("\n========================================\n");

rt_kprintf("  Sensor Monitoring (Press Ctrl+C to stop)\n");

rt_kprintf("  Refresh interval: %d ms\n", interval);

rt_kprintf("========================================\n\n");

while (1)

{

rt_kprintf("\033[2J\033[H"); /* Clear screen and move cursor to top-left */

rt_kprintf("========================================\n");

rt_kprintf("  Real-Time Sensor Monitoring\n");

rt_kprintf("========================================\n\n");

/* 火焰 */

rt_kprintf("Flame: A1=%4u A2=%4u A3=%4u A4=%4u A5=%4u\n",

g_adc_ch0, g_adc_ch1, g_adc_ch3, g_adc_ch4, g_adc_ch5);

/* 温湿度 */

int temp_int = (int)g_temperature_c;

int temp_dec = (int)((g_temperature_c - temp_int) * 100);

if (temp_dec < 0) temp_dec = -temp_dec;

int humi_int = (int)g_humidity_rh;

int humi_dec = (int)((g_humidity_rh - humi_int) * 100);

if (humi_dec < 0) humi_dec = -humi_dec;

rt_kprintf("Temp: %d.%02d°C  Humidity: %d.%02d%%RH\n",

temp_int, temp_dec, humi_int, humi_dec);

/* 氧气+甲烷 */

int o2_int = (int)g_o2_concentration;

int o2_dec = (int)((g_o2_concentration - o2_int) * 10);

if (o2_dec < 0) o2_dec = -o2_dec;

rt_kprintf("O2: %d.%d%%  Methane: %uppm (%u%%LEL)\n",

o2_int, o2_dec, g_methane_ppm, g_methane_lel);

/* 位移+应力 */

int disp_int = (int)Displacement;

int disp_dec = (int)((Displacement - disp_int) * 1000);

if (disp_dec < 0) disp_dec = -disp_dec;

rt_kprintf("Displacement: %d.%03dmm  ", disp_int, disp_dec);

if (g_stress_sensor_online && g_stress_value_valid)

{

int stress_int = (int)g_stress_value_n;

int stress_dec = (int)((g_stress_value_n - stress_int) * 100);

if (stress_dec < 0) stress_dec = -stress_dec;

rt_kprintf("Stress: %d.%02dN\n", stress_int, stress_dec);

}

else if (g_stress_sensor_online)

{

rt_kprintf("Stress: Invalid unit/status\n");

}

else

{

rt_kprintf("Stress: Offline\n");

}

/* 电流 */

int curr0 = (int)Current[0];

int curr1 = (int)Current[1];

int curr2 = (int)Current[2];

rt_kprintf("Current: P0=%dA P1=%dA P2=%dA\n", curr0, curr1, curr2);

rt_kprintf("\n[Press Ctrl+C to stop monitoring]\n");

rt_thread_mdelay(interval);

}

}

MSH_CMD_EXPORT_ALIAS(monitor_sensors, monitor, Monitor sensors in real-time);

/**

* @brief 测试单个传感器

*/

static void test_sensor(int argc, char **argv)

{

if (argc != 2)

{

rt_kprintf("\nUsage: test <sensor_name>\n");

rt_kprintf("\nAvailable sensors:\n");

rt_kprintf("  flame       - Five-channel flame sensor\n");

rt_kprintf("  sht30       - Temperature & humidity sensor\n");

rt_kprintf("  methane     - Methane gas sensor\n");

rt_kprintf("  o2          - Oxygen sensor\n");

rt_kprintf("  displacement - Displacement sensor\n");

rt_kprintf("  ammeter     - 3-phase ammeter\n");

rt_kprintf("\nExample: test flame\n\n");

return;

}

const char *sensor = argv[1];

if (rt_strcmp(sensor, "flame") == 0)

{

show_flame(0, NULL);

}

else if (rt_strcmp(sensor, "sht30") == 0)

{

rt_kprintf("\n[SHT30] Temperature: %.2f°C, Humidity: %.2f%%RH\n\n",

g_temperature_c, g_humidity_rh);

}

else if (rt_strcmp(sensor, "methane") == 0)

{

if (!g_methane_data_valid)

{

rt_kprintf("\n[Methane] No valid GM-402B frame received\n\n");

}

else

{

rt_kprintf("\n[Methane] Concentration: %uppm, LEL: %u%%, Status: 0x%02X%s\n\n"

,

g_methane_ppm, g_methane_lel, g_methane_status,

(g_methane_status == 0xAA) ? " (Normal)" : " (Check sensor)");

}

}

else if (rt_strcmp(sensor, "o2") == 0)

{

rt_kprintf("\n[O2] Concentration: %.1f%%\n\n", g_o2_concentration);

}

else if (rt_strcmp(sensor, "displacement") == 0)

{

if (!g_displacement_sensor_online)

{

rt_kprintf("\n[Displacement] Offline - no valid Modbus response\n\n");

return;

}

/* rt_kprintf in this build has no floating-point formatter.  Print

* the value as integer millimetres plus a three-digit fraction. */

int disp_int = (int)Displacement;

int disp_dec = (int)((Displacement - disp_int) * 1000.0f);

if (disp_dec < 0) disp_dec = -disp_dec;

rt_kprintf("\n[Displacement] Value: %d.%03dmm\n\n", disp_int, disp_dec);

}

else if (rt_strcmp(sensor, "ammeter") == 0)

{

rt_kprintf("\n[Ammeter]\n");

rt_kprintf("  Voltage: P0=%.2fV P1=%.2fV P2=%.2fV\n",

Voltage[0], Voltage[1], Voltage[2]);

rt_kprintf("  Current: P0=%.2fA P1=%.2fA P2=%.2fA\n\n",

Current[0], Current[1], Current[2]);

}

else

{

rt_kprintf("\nError: Unknown sensor '%s'\n", sensor);

rt_kprintf("Use 'test' without arguments to see available sensors.\n\n");

}

}

MSH_CMD_EXPORT_ALIAS(test_sensor, test, Test individual sensor);

/**

* @brief 显示 UART 引脚配置信息 - 已禁用，用于单独测试

*/

#if 0

static void show_uart_pins(int argc, char **argv)

{

rt_kprintf("\n========================================\n");

rt_kprintf("  UART Pin Configuration\n");

rt_kprintf("========================================\n");

rt_kprintf("\n");

rt_kprintf("UART1 (Console):\n");

rt_kprintf("  TX: PA9\n");

rt_kprintf("  RX: PA10\n");

rt_kprintf("\n");

rt_kprintf("UART2 (Methane Sensor GM-402B):\n");

rt_kprintf("  TX: PD5 (MCU -> Sensor RX)\n");

rt_kprintf("  RX: PD6 (MCU <- Sensor TX)\n");

rt_kprintf("  Baud: 19200\n");

rt_kprintf("  ** IMPORTANT: PA2/PA3 are NOT used! **\n");

rt_kprintf("  ** PA2 is used by ETH_MDIO **\n");

rt_kprintf("  ** PA3 is used by ADC (Flame A2) **\n");

rt_kprintf("\n");

rt_kprintf("UART3 (Modbus RTU):\n");

rt_kprintf("  TX: PB10\n");

rt_kprintf("  RX: PB11\n");

rt_kprintf("  Baud: 9600\n");

rt_kprintf("  Devices: Displacement, Stress, Ammeter\n");

rt_kprintf("\n");

rt_kprintf("UART4:\n");

rt_kprintf("  TX: PA12\n");

rt_kprintf("  RX: PA11\n");

rt_kprintf("\n");

rt_kprintf("UART5:\n");

rt_kprintf("  TX: PB6\n");

rt_kprintf("  RX: PB5\n");

rt_kprintf("\n");

rt_kprintf("========================================\n");

rt_kprintf("Hardware Connection Checklist:\n");

rt_kprintf("========================================\n");

rt_kprintf("[ ] GM-402B TX -> MCU PD6 (UART2_RX)\n");

rt_kprintf("[ ] GM-402B RX -> MCU PD5 (UART2_TX)\n");

rt_kprintf("[ ] GM-402B GND -> MCU GND\n");

rt_kprintf("[ ] GM-402B VIN -> 3.3V or 5V\n");

rt_kprintf("[ ] Verify sensor power LED is ON\n");

rt_kprintf("========================================\n\n");

}

#endif

/* MSH_CMD_EXPORT_ALIAS(show_uart_pins, uart_pins, Show UART pin configuration); */

/**

* @brief 检查 UART2 实际的引脚配置（从寄存器读取）- 已禁用，用于单独测试

*/

#if 0

static void check_uart2_pins(int argc, char **argv)

{

rt_kprintf("\n========================================\n");

rt_kprintf("  UART2 Actual Pin Configuration\n");

rt_kprintf("========================================\n");

rt_kprintf("\n");

/* USART2 寄存器基址 */

USART_TypeDef *uart2 = USART2;

rt_kprintf("USART2 Hardware Info:\n");

rt_kprintf("  Base Address: 0x%08X\n", (unsigned int)uart2);

rt_kprintf("  CR1 Register: 0x%08X\n", (unsigned int)uart2->CR1);

rt_kprintf("  Enabled: %s\n", (uart2->CR1 & USART_CR1_UE) ? "YES" : "NO");

rt_kprintf("  TX Enabled: %s\n", (uart2->CR1 & USART_CR1_TE) ? "YES" : "NO");

rt_kprintf("  RX Enabled: %s\n", (uart2->CR1 & USART_CR1_RE) ? "YES" : "NO");

rt_kprintf("\n");

rt_kprintf("Expected Pin Configuration:\n");

rt_kprintf("  TX: PD5 (GPIOD Pin 5)\n");

rt_kprintf("  RX: PD6 (GPIOD Pin 6)\n");

rt_kprintf("\n");

/* 检查 GPIOD 的 AFR (Alternate Function Register) */

GPIO_TypeDef *gpiod = GPIOD;

rt_kprintf("GPIOD Actual Configuration:\n");

/* PD5 配置 */

rt_uint32_t moder5 = (gpiod->MODER >> (5 * 2)) & 0x3;

rt_uint32_t afr5 = (gpiod->AFR[0] >> (5 * 4)) & 0xF;

rt_kprintf("  PD5 (UART2_TX):\n");

rt_kprintf("    Mode: 0x%X ", moder5);

if (moder5 == 0) rt_kprintf("(Input)");

else if (moder5 == 1) rt_kprintf("(Output)");

else if (moder5 == 2) rt_kprintf("(Alternate Function)");

else if (moder5 == 3) rt_kprintf("(Analog)");

rt_kprintf("\n");

rt_kprintf("    AF: AF%u", afr5);

if (afr5 == 7) rt_kprintf(" (USART2 - Correct!)");

else rt_kprintf(" (Wrong! Should be AF7)");

rt_kprintf("\n");

/* PD6 配置 */

rt_uint32_t moder6 = (gpiod->MODER >> (6 * 2)) & 0x3;

rt_uint32_t afr6 = (gpiod->AFR[0] >> (6 * 4)) & 0xF;

rt_kprintf("  PD6 (UART2_RX):\n");

rt_kprintf("    Mode: 0x%X ", moder6);

if (moder6 == 0) rt_kprintf("(Input)");

else if (moder6 == 1) rt_kprintf("(Output)");

else if (moder6 == 2) rt_kprintf("(Alternate Function)");

else if (moder6 == 3) rt_kprintf("(Analog)");

rt_kprintf("\n");

rt_kprintf("    AF: AF%u", afr6);

if (afr6 == 7) rt_kprintf(" (USART2 - Correct!)");

else rt_kprintf(" (Wrong! Should be AF7)");

rt_kprintf("\n");

rt_kprintf("\n");

rt_kprintf("Diagnosis:\n");

if (moder5 == 2 && afr5 == 7 && moder6 == 2 && afr6 == 7)

{

rt_kprintf("  ✓ UART2 is correctly mapped to PD5/PD6\n");

}

else

{

rt_kprintf("  ✗ UART2 pin configuration is WRONG!\n");

if (moder5 != 2 || afr5 != 7)

rt_kprintf("  ✗ PD5 is not configured as UART2_TX\n");

if (moder6 != 2 || afr6 != 7)

rt_kprintf("  ✗ PD6 is not configured as UART2_RX\n");

}

rt_kprintf("\n========================================\n\n");

}

#endif

/* MSH_CMD_EXPORT_ALIAS(check_uart2_pins, check_uart2, Check UART2 actual pin configuratio

n from registers); */

/**

* @brief O2 sensor debug command - shows raw ADC and O2 concentration

*/

static void o2_debug(int argc, char **argv)

{

rt_adc_device_t adc_dev;

rt_uint32_t raw;

rt_int32_t o2_x10;

float o2_percent;

rt_kprintf("\n========================================\n");

rt_kprintf("  O2 Sensor Debug (ADC1 Channel 18)\n");

rt_kprintf("========================================\n\n");

/* Find ADC device */

adc_dev = (rt_adc_device_t)rt_device_find("adc1");

if (adc_dev == RT_NULL)

{

rt_kprintf("ERROR: Cannot find adc1 device!\n\n");

return;

}

/* Enable ADC */

rt_adc_enable(adc_dev, 18);

/* Read raw ADC value */

raw = rt_adc_read(adc_dev, 18);

/* Convert to O2 percentage (x10) using ADC raw value directly */

o2_x10 = (rt_int32_t)(raw - 0) * 209 / (2410 - 0);

/* Air-stabilize */

if (o2_x10 >= 207 && o2_x10 <= 211)

{

o2_x10 = 209;

}

o2_percent = (float)o2_x10 / 10.0f;

rt_kprintf("Raw ADC Value:     %u (0-65535)\n", raw);

rt_kprintf("O2 Concentration:  %.1f %%\n", o2_percent);

rt_kprintf("\n");

rt_kprintf("Calibration Parameters:\n");

rt_kprintf("  Zero Offset:     0 (ADC raw value)\n");

rt_kprintf("  Full Scale:      2410 (ADC raw at 20.9%% O2)\n");

rt_kprintf("  Stabilize Range: 20.7%% - 21.1%% -> 20.9%%\n");

rt_kprintf("\n");

rt_kprintf("Expected Values:\n");

rt_kprintf("  Normal Air:      ADC ~2410, O2 ~20.9%%\n");

rt_kprintf("  Low O2 Warning:  <19.5%%\n");

rt_kprintf("  High O2 Warning: >23.5%%\n");

rt_kprintf("\n");

if (raw < 500)

{

rt_kprintf("WARNING: ADC value very low!\n");

rt_kprintf("  Check sensor power and connections.\n");

}

else if (o2_percent < 10.0f || o2_percent > 30.0f)

{

rt_kprintf("WARNING: O2 value out of normal range!\n");

rt_kprintf("  Possible causes:\n");

rt_kprintf("  1. Sensor needs calibration (adjust O2_FULL_SCALE_ADC)\n");

rt_kprintf("  2. Current calibration: ADC 2410 = 20.9%% O2\n");

rt_kprintf("  3. Your ADC: %u -> %.1f%% O2\n", raw, o2_percent);

}

rt_kprintf("========================================\n\n");

rt_adc_disable(adc_dev, 18);

}

MSH_CMD_EXPORT_ALIAS(o2_debug, o2_debug, Debug O2 sensor raw ADC and concentration);

// File: applications/Apps/debugCommands.h

/*

* Copyright (c) 2006-2026, RT-Thread Development Team

*

* SPDX-License-Identifier: Apache-2.0

*

* Change Logs:

* Date           Author       Notes

* 2026-09-18     ideapad15s   Debug MSH commands for sensor testing

*/

#ifndef APPLICATIONS_APPS_DEBUGCOMMANDS_H_

#define APPLICATIONS_APPS_DEBUGCOMMANDS_H_

/* MSH命令已通过MSH_CMD_EXPORT导出，无需额外函数声明 */

/*

* 可用的MSH调试命令：

*

* sensors          - 显示所有传感器数据汇总

* flame            - 显示火焰传感器详细状态

* env              - 显示环境传感器（温湿度、氧气、甲烷）

* modbus           - 显示Modbus设备状态

* monitor [sec]    - 实时监控传感器数据（可选刷新间隔秒数）

* test <sensor>    - 测试单个传感器

*

*/

#endif /* APPLICATIONS_APPS_DEBUGCOMMANDS_H_ */

// File: applications/Apps/displacementSensorApp.c

/*

* Copyright (c) 2006-2021, RT-Thread Development Team

*

* SPDX-License-Identifier: Apache-2.0

*

* Change Logs:

* Date           Author       Notes

* 2026-08-31     Administrator       Displacement sensor application

*/

#include "displacementSensorApp.h"

#include "freeModbusApp.h"

/* 全局变量定义 */

float g_displacement_value = 0.0f;

rt_bool_t g_displacement_sensor_online = RT_FALSE;

/**

* @brief 打印位移传感器状态信息

*/

void displacement_print_status(void)

{

if (g_displacement_sensor_online)

{

int disp_int = (int)g_displacement_value;

int disp_dec = (int)((g_displacement_value - disp_int) * 1000);

if (disp_dec < 0) disp_dec = -disp_dec;

rt_kprintf("[Displacement] Online - Value: %d.%03d mm\n", disp_int, disp_dec);

}

else

{

rt_kprintf("[Displacement] Offline - No communication\n");

}

}

/**

* @brief 位移传感器测试线程入口

*

* 该线程用于独立测试位移传感器的Modbus通信

* 在确认通信正常后，再将其集成到主Modbus轮询线程中

*

* @param parameter 线程参数(未使用)

*/

void displacement_sensor_test_thread_entry(void *parameter)

{

eMBMasterReqErrCode error_code = MB_MRE_NO_ERR;

uint16_t reg_low;

uint32_t success_count = 0;

uint32_t error_count = 0;

rt_kprintf("\n========================================\n");

rt_kprintf("  Displacement Sensor Test Started\n");

rt_kprintf("========================================\n");

rt_kprintf("  Slave Address: %d\n", DISPLACEMENT_SLAVE_ADDR);

rt_kprintf("  Register Start: 0x%04X\n", DISPLACEMENT_REG_START);

rt_kprintf("  Register Count: %d\n", DISPLACEMENT_REG_NUM);

rt_kprintf("  Baudrate: 9600\n");

rt_kprintf("  Parity: None\n");

rt_kprintf("  Scale Factor: %.3f (1 unit = 0.025mm)\n", DISPLACEMENT_SCALE_FACTOR);

rt_kprintf("========================================\n\n");

/* 等待Modbus主站初始化完成 */

rt_thread_mdelay(2000);

while (1)

{

/* 读取位移传感器的保持寄存器 */

error_code = eMBMasterReqReadHoldingRegister(

DISPLACEMENT_SLAVE_ADDR,

DISPLACEMENT_REG_START,

DISPLACEMENT_REG_NUM,

RT_WAITING_FOREVER

);

if (error_code == MB_MRE_NO_ERR)

{

/* 从缓冲区读取寄存器值 (从站地址1对应缓冲区索引0) */

reg_low  = usMRegHoldBuf[DISPLACEMENT_SLAVE_ADDR - 1][DISPLACEMENT_REG_START +

1];

g_displacement_sensor_online = RT_TRUE;

success_count++;

/* 位移传感器数据只在 Reg[1] 中 */

uint16_t raw_value = reg_low;

/* 减去零点偏移，然后转换为mm */

int32_t adjusted_raw = (int32_t)raw_value - DISPLACEMENT_ZERO_OFFSET;

if (adjusted_raw < 0) adjusted_raw = 0;  /* 防止负值 */

g_displacement_value = adjusted_raw * DISPLACEMENT_SCALE_FACTOR;

/* 打印简洁的数据 */

int disp_int = (int)g_displacement_value;

int disp_dec = (int)((g_displacement_value - disp_int) * 1000);

if (disp_dec < 0) disp_dec = -disp_dec;

rt_kprintf("[%05d] Displacement: %d.%03d mm (Raw: %d) | OK: %d, ERR: %d\n",

success_count + error_count,

disp_int, disp_dec,

raw_value,

success_count, error_count);

}

else

{

/* 通信错误处理 */

g_displacement_sensor_online = RT_FALSE;

error_count++;

rt_kprintf("[%05d] Displacement Error: Code=%d | OK: %d, ERR: %d\n",

success_count + error_count,

error_code,

success_count, error_count);

}

/* 每1秒读取一次 */

rt_thread_mdelay(1000);

}

}

/**

* @brief 修改位移传感器的Modbus从站地址

*

* @param old_addr 当前从站地址

* @param new_addr 新的从站地址（1-247）

* @return RT_EOK 成功，-RT_ERROR 失败

*/

rt_err_t displacement_change_slave_address(uint8_t old_addr, uint8_t new_addr)

{

eMBMasterReqErrCode error_code;

USHORT reg_value = (USHORT)new_addr;

rt_kprintf("\n========================================\n");

rt_kprintf("  Displacement Sensor Address Change\n");

rt_kprintf("========================================\n");

rt_kprintf("  Old Address: %d (0x%02X)\n", old_addr, old_addr);

rt_kprintf("  New Address: %d (0x%02X)\n", new_addr, new_addr);

rt_kprintf("  Register: 0x0004\n");

rt_kprintf("  Baudrate: 9600, 8N1\n");

rt_kprintf("  Modbus Frame: %02X 06 00 04 00 %02X [CRC]\n", old_addr, new_addr);

rt_kprintf("========================================\n\n");

rt_kprintf("[DISP] Sending address change command...\n");

/* 写从站地址配置寄存器 (0x0004) */

error_code = eMBMasterReqWriteHoldingRegister(

old_addr,           // 使用旧地址发送命令

0x0004,             // 从站地址配置寄存器

reg_value,          // 新地址值

RT_WAITING_FOREVER  // 等待响应

);

if (error_code == MB_MRE_NO_ERR)

{

rt_kprintf("[DISP] ✓ Address changed successfully!\n");

rt_kprintf("[DISP] ✓ New address: %d (0x%02X)\n", new_addr, new_addr);

rt_kprintf("[DISP] ✓ Change is permanent (saved in non-volatile memory)\n");

rt_kprintf("[DISP] ! Please run 'disp_test_addr %d' to verify\n", new_addr);

return RT_EOK;

}

else

{

rt_kprintf("[DISP] ✗ Failed to change address\n");

rt_kprintf("[DISP] ✗ Error code: %d\n", error_code);

rt_kprintf("[DISP] ! Check connections and ensure no other device uses address %d\

n", old_addr);

return -RT_ERROR;

}

}

/**

* @brief 使用指定地址测试读取位移值

*/

static void displacement_test_address(uint8_t test_addr)

{

eMBMasterReqErrCode error_code;

uint16_t raw_value;

rt_kprintf("\n[DISP] Testing address %d...\n", test_addr);

error_code = eMBMasterReqReadHoldingRegister(

test_addr,          // 测试地址

0x0000,             // 位移值寄存器

2,                  // 读取2个寄存器

RT_WAITING_FOREVER

);

if (error_code == MB_MRE_NO_ERR)

{

raw_value = usMRegHoldBuf[test_addr - 1][1];

int32_t adjusted = (int32_t)raw_value - 74;

if (adjusted < 0) adjusted = 0;

/* 0.025 mm per raw count = 25 thousandths of a millimetre. */

int32_t displacement_milli = adjusted * 25;

rt_kprintf("[DISP] ✓ Address %d responds correctly!\n", test_addr);

rt_kprintf("[DISP]   Raw value: %d\n", raw_value);

rt_kprintf("[DISP]   Displacement: %d.%03d mm\n",

(int)(displacement_milli / 1000),

(int)(displacement_milli % 1000));

}

else

{

rt_kprintf("[DISP] ✗ Address %d test failed\n", test_addr);

rt_kprintf("[DISP] ✗ Error code: %d\n", error_code);

rt_kprintf("[DISP] ! Device may not be at this address\n");

}

}

/**

* @brief MSH命令：修改位移传感器地址

* 使用方法: disp_set_addr 1 2

*/

static void disp_set_addr(int argc, char **argv)

{

uint8_t old_addr, new_addr;

if (argc != 3)

{

rt_kprintf("\nUsage: disp_set_addr <old_addr> <new_addr>\n");

rt_kprintf("Example: disp_set_addr 1 2\n");

rt_kprintf("         (Change from address 1 to address 2)\n\n");

return;

}

old_addr = atoi(argv[1]);

new_addr = atoi(argv[2]);

if (old_addr < 1 || old_addr > 247 || new_addr < 1 || new_addr > 247)

{

rt_kprintf("\nError: Address must be in range 1-247\n\n");

return;

}

if (old_addr == new_addr)

{

rt_kprintf("\nError: Old and new addresses are the same!\n\n");

return;

}

rt_kprintf("\n⚠️  WARNING: Ensure all other Modbus devices are disconnected!\n");

rt_kprintf("⚠️  WARNING: Only the displacement sensor should be on the bus!\n\n");

displacement_change_slave_address(old_addr, new_addr);

}

MSH_CMD_EXPORT(disp_set_addr, Change displacement sensor slave address);

/**

* @brief MSH命令：测试位移传感器地址

* 使用方法: disp_test_addr 2

*/

static void disp_test_addr(int argc, char **argv)

{

uint8_t test_addr;

if (argc != 2)

{

rt_kprintf("\nUsage: disp_test_addr <address>\n");

rt_kprintf("Example: disp_test_addr 2\n");

rt_kprintf("         (Test if device responds at address 2)\n\n");

return;

}

test_addr = atoi(argv[1]);

if (test_addr < 1 || test_addr > 247)

{

rt_kprintf("\nError: Address must be in range 1-247\n\n");

return;

}

displacement_test_address(test_addr);

}

MSH_CMD_EXPORT(disp_test_addr, Test displacement sensor address);

// File: applications/Apps/displacementSensorApp.h

/*

* Copyright (c) 2006-2021, RT-Thread Development Team

*

* SPDX-License-Identifier: Apache-2.0

*

* Change Logs:

* Date           Author       Notes

* 2026-08-31     Administrator       Displacement sensor application

*/

#ifndef APPLICATIONS_APPS_DISPLACEMENTSENSORAPP_H_

#define APPLICATIONS_APPS_DISPLACEMENTSENSORAPP_H_

#include "heads.h"

/*************************** Displacement Sensor MODBUS ****************************/

/*

* 拉绳位移传感器通信参数（基于PDF文档）

* - 通信协议: Modbus RTU

* - 波特率: 9600 (默认)

* - 数据位: 8

* - 停止位: 1

* - 校验位: None

* - 从站地址: 2 (已修改，原默认为0x01)

*/

#define DISPLACEMENT_SLAVE_ADDR         2           /* 位移传感器从站地址 (已修改为2) */

#define DISPLACEMENT_REG_START          0x0000      /* 位移值寄存器起始地址 */

#define DISPLACEMENT_REG_NUM            2           /* 位移值占用2个寄存器(32位浮点数) */

/* 位移传感器功能码 */

#define DISPLACEMENT_FUNC_READ          0x03        /* 读保持寄存器 */

#define DISPLACEMENT_FUNC_WRITE         0x10        /* 写多个寄存器 */

/* 位移传感器寄存器地址定义（基于PDF文档） */

#define DISPLACEMENT_VALUE_REG          0x0000      /* 当前位移值 (16bit integer in Reg[1]

) */

#define DISPLACEMENT_SLAVE_ADDR_REG     0x0100      /* 从站地址设置寄存器 */

#define DISPLACEMENT_BAUDRATE_REG       0x0101      /* 波特率设置寄存器 */

/* 位移传感器数据换算系数 */

#define DISPLACEMENT_SCALE_FACTOR       0.025f      /* 寄存器值转换为mm的系数 (1单位 = 0.0

25mm) */

/* 实测：1cm = ~400单位，即 1单位 = 0

.025mm */

#define DISPLACEMENT_ZERO_OFFSET        74          /* 零点偏移量（原位时的寄存器值） */

/* 实测：拉线完全收回时 Raw ≈ 74 */

/* 全局变量 */

extern float g_displacement_value;                  /* 当前位移值 (mm 或配置的单位) */

extern rt_bool_t g_displacement_sensor_online;      /* 传感器在线状态 */

/* 线程入口函数 */

void displacement_sensor_test_thread_entry(void *parameter);

/* 工具函数 */

void displacement_print_status(void);

#endif /* APPLICATIONS_APPS_DISPLACEMENTSENSORAPP_H_ */

// File: applications/Apps/ethApp.c

﻿/*

* Copyright (c) 2006-2026, RT-Thread Development Team

*

* SPDX-License-Identifier: Apache-2.0

*

* Change Logs:

* Date           Author       Notes

* 2026-06-18     ideapad15s   Ethernet TCP client for fiber optic link

*/

#include "ethApp.h"

#include "huaweiCloudApp.h"

#include "sht30App.h"

#include "o2SensorApp.h"

#include "linesensor.h"

#include "stressSensorApp.h"

#include <rtthread.h>

#include <sys/socket.h>

#include <netdb.h>

#include <string.h>

#include <stdio.h>

#define DBG_TAG "EthApp"

#define DBG_LVL DBG_ERROR

#include <rtdbg.h>

static char eth_json_buf[512];

static int  eth_sock = -1;

static int append_float(char *buf, int size, const char *key, float val)

{

int i = (int)val;

int d = (int)((val - i) * 100);

if (d < 0) d = -d;

return rt_snprintf(buf, size, "\"%s\":%d.%02d", key, i, d);

}

static int eth_tcp_connect(void)

{

struct sockaddr_in server_addr;

int sock;

sock = socket(AF_INET, SOCK_STREAM, 0);

if (sock < 0)

{

LOG_E("socket create failed");

return -1;

}

server_addr.sin_family = AF_INET;

server_addr.sin_port = htons(ETH_SERVER_PORT);

server_addr.sin_addr.s_addr = inet_addr(ETH_SERVER_IP);

rt_memset(&(server_addr.sin_zero), 0, sizeof(server_addr.sin_zero));

if (connect(sock, (struct sockaddr *)&server_addr,

sizeof(struct sockaddr)) < 0)

{

LOG_E("connect to %s:%d failed", ETH_SERVER_IP, ETH_SERVER_PORT);

closesocket(sock);

return -1;

}

LOG_I("connected to %s:%d", ETH_SERVER_IP, ETH_SERVER_PORT);

return sock;

}

static int build_sensor_json(void)

{

int pos;

pos = rt_snprintf(eth_json_buf, sizeof(eth_json_buf),

"{\"device_id\":\"%s\",\"data\":{"

"\"ch0\":%u,\"ch1\":%u,\"ch3\":%u,\"ch4\":%u,\"ch5\":%u,"

"\"methane_ppm\":%u,\"methane_lel\":%u,",

ETH_DEVICE_ID,

(unsigned int)g_adc_ch0,

(unsigned int)g_adc_ch1,

(unsigned int)g_adc_ch3,

(unsigned int)g_adc_ch4,

(unsigned int)g_adc_ch5,

(unsigned int)g_methane_ppm,

(unsigned int)g_methane_lel);

pos += append_float(eth_json_buf + pos,

sizeof(eth_json_buf) - pos, "voltage_a", Voltage[0]);

pos += rt_snprintf(eth_json_buf + pos,

sizeof(eth_json_buf) - pos, ",");

pos += append_float(eth_json_buf + pos,

sizeof(eth_json_buf) - pos, "voltage_b", Voltage[1]);

pos += rt_snprintf(eth_json_buf + pos,

sizeof(eth_json_buf) - pos, ",");

pos += append_float(eth_json_buf + pos,

sizeof(eth_json_buf) - pos, "voltage_c", Voltage[2]);

pos += rt_snprintf(eth_json_buf + pos,

sizeof(eth_json_buf) - pos, ",");

pos += append_float(eth_json_buf + pos,

sizeof(eth_json_buf) - pos, "current_a", Current[0]);

pos += rt_snprintf(eth_json_buf + pos,

sizeof(eth_json_buf) - pos, ",");

pos += append_float(eth_json_buf + pos,

sizeof(eth_json_buf) - pos, "current_b", Current[1]);

pos += rt_snprintf(eth_json_buf + pos,

sizeof(eth_json_buf) - pos, ",");

pos += append_float(eth_json_buf + pos,

sizeof(eth_json_buf) - pos, "current_c", Current[2]);

pos += rt_snprintf(eth_json_buf + pos,

sizeof(eth_json_buf) - pos, ",");

pos += append_float(eth_json_buf + pos,

sizeof(eth_json_buf) - pos, "flow", Flow);

pos += rt_snprintf(eth_json_buf + pos,

sizeof(eth_json_buf) - pos, ",");

pos += append_float(eth_json_buf + pos,

sizeof(eth_json_buf) - pos, "temperature", g_temperature_c);

pos += rt_snprintf(eth_json_buf + pos,

sizeof(eth_json_buf) - pos, ",");

pos += append_float(eth_json_buf + pos,

sizeof(eth_json_buf) - pos, "humidity", g_humidity_rh);

pos += rt_snprintf(eth_json_buf + pos,

sizeof(eth_json_buf) - pos, ",");

pos += append_float(eth_json_buf + pos,

sizeof(eth_json_buf) - pos, "o2", g_o2_concentration);

pos += rt_snprintf(eth_json_buf + pos,

sizeof(eth_json_buf) - pos, ",");

pos += append_float(eth_json_buf + pos,

sizeof(eth_json_buf) - pos, "displacement", Displacement);

pos += rt_snprintf(eth_json_buf + pos,

sizeof(eth_json_buf) - pos, ",");

pos += append_float(eth_json_buf + pos,

sizeof(eth_json_buf) - pos, "stress", g_stress_value_n);

pos += rt_snprintf(eth_json_buf + pos,

sizeof(eth_json_buf) - pos,

",\"flame\":%d}}\n", (int)Flame);

return pos;

}

static void eth_client_thread_entry(void *parameter)

{

int len, ret;

int fail_count = 0;

LOG_I("Ethernet TCP client thread started.");

rt_thread_mdelay(8000);

while (1)

{

if (eth_sock < 0)

{

eth_sock = eth_tcp_connect();

if (eth_sock < 0)

{

rt_thread_mdelay(ETH_RECONNECT_DELAY);

continue;

}

fail_count = 0;

}

len = build_sensor_json();

ret = send(eth_sock, eth_json_buf, len, 0);

if (ret <= 0)

{

fail_count++;

LOG_W("TCP send failed (%d/3)", fail_count);

if (fail_count >= 3)

{

LOG_E("Too many failures, reconnecting...");

closesocket(eth_sock);

eth_sock = -1;

}

}

else

{

fail_count = 0;

LOG_D("Sent %d bytes via ETH", ret);

}

rt_thread_mdelay(ETH_REPORT_INTERVAL);

}

}

rt_err_t eth_app_init(void)

{

rt_thread_t tid;

tid = rt_thread_create("eth_tcp",

eth_client_thread_entry,

RT_NULL,

2048,

24,

10);

if (tid == RT_NULL)

{

LOG_E("Failed to create ETH TCP thread!");

return -RT_ERROR;

}

rt_thread_startup(tid);

LOG_I("ETH TCP client thread created.");

return RT_EOK;

}

// File: applications/Apps/ethApp.h

/*

* Copyright (c) 2006-2026, RT-Thread Development Team

*

* SPDX-License-Identifier: Apache-2.0

*

* Change Logs:

* Date           Author       Notes

* 2026-06-18     ideapad15s   Ethernet TCP client for fiber optic link

*/

#ifndef APPLICATIONS_APPS_ETHAPP_H_

#define APPLICATIONS_APPS_ETHAPP_H_

#include <rtthread.h>

/* TCP server config */

#define ETH_SERVER_IP           "192.168.3.10"

#define ETH_SERVER_PORT         19008

/* Device identification */

#define ETH_DEVICE_ID           "pipe_gallery_node_01"

/* Report interval (ms) */

#define ETH_REPORT_INTERVAL     3000

/* Reconnect delay (ms) */

#define ETH_RECONNECT_DELAY     5000

/**

* @brief Initialize ethernet TCP client and start report thread

* @return RT_EOK on success

*/

rt_err_t eth_app_init(void);

#endif /* APPLICATIONS_APPS_ETHAPP_H_ */

// File: applications/Apps/freeModbusApp.c

﻿/*

* Copyright (c) 2006-2021, RT-Thread Development Team

*

* SPDX-License-Identifier: Apache-2.0

*

* Change Logs:

* Date           Author       Notes

* 2025-01-18     Administrator       the first version

*/

#include "freeModbusApp.h"

#include "displacementSensorApp.h"

#include "stressSensorApp.h"

/* global variables */

float Voltage[3] = {0};     /* three-phase voltage */

float Current[3] = {0};     /* three-phase current */

float Flow = 0;             /* water flow */

float Displacement = 0;     /* displacement value (mm) */

/**

* @brief Convert two 16-bit registers to an IEEE 754 float

*

* @param reg1 high 16-bit register value

* @param reg2 low 16-bit register value

* @return float converted float value

*

* Conversion steps:

* 1. treat reg2 as unsigned 16-bit value

* 2. combine reg1 as high 16 bits and reg2 as low 16 bits into one 32-bit value

* 3. reinterpret the 32-bit bit pattern as IEEE 754 float

*/

static float registers_to_float(uint16_t reg1, uint16_t reg2)

{

uint32_t combined;

float result;

/* step 1: make reg2 unsigned */

uint16_t reg2_unsigned = reg2 & 0xFFFF;

/* step 2: combine into one 32-bit value, high word first */

combined = ((uint32_t)reg1 << 16) | reg2_unsigned;

/* step 3: reinterpret as IEEE 754 float */

memcpy(&result, &combined, sizeof(float));

return result;

}

/* Ammeter voltage query: 49 03 01 00 00 06 CB BC */

/* Ammeter current query: 49 03 01 0E 00 06 AA 7F */

/* Water meter flow query: 01 03 00 01 00 02 95 CB */

/* (water meter signal line: A and B) */

void send_thread_entry(void *parameter)

{

eMBMasterReqErrCode error_code = MB_MRE_NO_ERR;

uint16_t reg_high, reg_low;

while (1)

{

/* ==================== Read water meter flow ==================== */

error_code = eMBMasterReqReadHoldingRegister(WATERMETER_SLAVE_ADDR,

FLOW_REG_START,

FLOW_REG_NUM,

RT_WAITING_FOREVER);

if (error_code == MB_MRE_NO_ERR)

{

/* read registers from buffer: slave address 1 corresponds to buffer index 0 *

/

reg_high = usMRegHoldBuf[WATERMETER_SLAVE_ADDR - 1][FLOW_REG_START];

reg_low  = usMRegHoldBuf[WATERMETER_SLAVE_ADDR - 1][FLOW_REG_START + 1];

/* convert to float */

Flow = registers_to_float(reg_high, reg_low);

/* update node data */

node[0].Flow = Flow;

int flow_int = (int)Flow;

int flow_dec = (int)((Flow - flow_int) * 1000);

if (flow_dec < 0) flow_dec = -flow_dec;

/* rt_kprintf("[Water] Flow: %d.%03d m3/h (Reg[%d]=0x%04X, Reg[%d]=0x%04X)\n",

flow_int, flow_dec, FLOW_REG_START, reg_high, FLOW_REG_START+1, reg

_low); */

}

else

{

/* rt_kprintf("[Water] Error: %d\n", error_code); */

}

rt_thread_mdelay(500);

/* ==================== Read displacement sensor ==================== */

error_code = eMBMasterReqReadHoldingRegister(DISPLACEMENT_SLAVE_ADDR,

DISPLACEMENT_REG_START,

DISPLACEMENT_REG_NUM,

RT_WAITING_FOREVER);

if (error_code == MB_MRE_NO_ERR)

{

/* Displacement sensor data is in Reg[1] only */

uint16_t raw_value = usMRegHoldBuf[DISPLACEMENT_SLAVE_ADDR - 1][DISPLACEMENT_R

EG_START + 1];

/* Apply zero offset and convert to mm */

int32_t adjusted_raw = (int32_t)raw_value - 298;  /* Zero offset = 298 (adjust

ed from 74 to compensate 5mm error) */

if (adjusted_raw < 0) adjusted_raw = 0;

Displacement = adjusted_raw * 0.025f;  /* Scale factor = 0.025 */

g_displacement_value = Displacement;

g_displacement_sensor_online = RT_TRUE;

/* Update node data */

node[0].Displacement = Displacement;

/* Optional: print debug info */

// int disp_int = (int)Displacement;

// int disp_dec = (int)((Displacement - disp_int) * 1000);

// if (disp_dec < 0) disp_dec = -disp_dec;

// rt_kprintf("[Displacement] Value: %d.%03d mm (Raw: %d)\n", disp_int, disp_d

ec, raw_value);

}

else

{

g_displacement_sensor_online = RT_FALSE;

/* rt_kprintf("[Displacement] Error: %d\n", error_code); */

}

rt_thread_mdelay(500);

/* ==================== Read RDF-TC25 stress sensor ==================== */

(void)stress_sensor_poll();

rt_thread_mdelay(500);

/* ==================== Read ammeter voltage ==================== */

error_code = eMBMasterReqReadHoldingRegister(AMMETER_SLAVE_ADDR,

VOLTAGE_REG_START,

VOLTAGE_REG_NUM,

RT_WAITING_FOREVER);

if (error_code == MB_MRE_NO_ERR)

{

/* slave address 73 requires MB_MASTER_TOTAL_SLAVE_NUM >= 73 */

for (int i = 0; i < 3; i++)

{

reg_high = usMRegHoldBuf[AMMETER_SLAVE_ADDR - 1][VOLTAGE_REG_START + i * 2

];

reg_low  = usMRegHoldBuf[AMMETER_SLAVE_ADDR - 1][VOLTAGE_REG_START + i * 2

+ 1];

Voltage[i] = registers_to_float(reg_high, reg_low);

int volt_int = (int)Voltage[i];

int volt_dec = (int)((Voltage[i] - volt_int) * 100);

if (volt_dec < 0) volt_dec = -volt_dec;

/* rt_kprintf("[Ammeter] Voltage[%d]: %d.%02d V (Reg[0x%03X]=0x%04X, Reg[0

x%03X]=0x%04X)\n",

i, volt_int, volt_dec,

VOLTAGE_REG_START + i*2, reg_high,

VOLTAGE_REG_START + i*2 + 1, reg_low); */

}

}

else

{

/* rt_kprintf("[Ammeter Voltage] Error: %d\n", error_code); */

}

rt_thread_mdelay(500);

/* ==================== Read ammeter current ==================== */

error_code = eMBMasterReqReadHoldingRegister(AMMETER_SLAVE_ADDR,

CURRENT_REG_START,

CURRENT_REG_NUM,

RT_WAITING_FOREVER);

if (error_code == MB_MRE_NO_ERR)

{

for (int i = 0; i < 3; i++)

{

reg_high = usMRegHoldBuf[AMMETER_SLAVE_ADDR - 1][CURRENT_REG_START + i * 2

];

reg_low  = usMRegHoldBuf[AMMETER_SLAVE_ADDR - 1][CURRENT_REG_START + i * 2

+ 1];

Current[i] = registers_to_float(reg_high, reg_low);

int curr_int = (int)Current[i];

int curr_dec = (int)((Current[i] - curr_int) * 1000);

if (curr_dec < 0) curr_dec = -curr_dec;

/* rt_kprintf("[Ammeter] Current[%d]: %d.%03d A (Reg[0x%03X]=0x%04X, Reg[0

x%03X]=0x%04X)\n",

i, curr_int, curr_dec,

CURRENT_REG_START + i*2, reg_high,

CURRENT_REG_START + i*2 + 1, reg_low); */

}

/* update node current field */

memcpy(node[0].CH1_A, Current, sizeof(Current));

}

else

{

/* rt_kprintf("[Ammeter Current] Error: %d\n", error_code); */

}

/* rt_kprintf("\n"); */  /* print separator line */

rt_thread_mdelay(500);

}

}

void mb_master_poll(void *parameter)

{

eMBMasterInit(MB_RTU, PORT_NUM, PORT_BAUDRATE, PORT_PARITY);

eMBMasterEnable();

while (1)

{

eMBMasterPoll();

rt_thread_mdelay(MB_POLL_CYCLE_MS);

}

}

// File: applications/Apps/freeModbusApp.h

/*

* Copyright (c) 2006-2021, RT-Thread Development Team

*

* SPDX-License-Identifier: Apache-2.0

*

* Change Logs:

* Date           Author       Notes

* 2025-01-16     Administrator       the first version

*/

#ifndef APPLICATIONS_APPS_FREEMODBUSAPP_H_

#define APPLICATIONS_APPS_FREEMODBUSAPP_H_

#include "heads.h"

extern USHORT   usMRegInBuf[MB_MASTER_TOTAL_SLAVE_NUM][M_REG_INPUT_NREGS];

extern USHORT   usMRegHoldBuf[MB_MASTER_TOTAL_SLAVE_NUM][M_REG_HOLDING_NREGS];

#define SLAVE_ADDR      1

#define PORT_NUM        3       /* use UART3 for Modbus communication */

#define PORT_BAUDRATE   9600

#define PORT_PARITY     MB_PAR_NONE

#define MB_POLL_THREAD_PRIORITY  10

#define MB_SEND_THREAD_PRIORITY  RT_THREAD_PRIORITY_MAX - 1

#define MB_SEND_REG_START  1

#define MB_SEND_REG_NUM    10

#define MB_READ_REG_START  0

#define MB_READ_NUM        10

#define MODBUS_NUM   1

/*************************** Ammeter MODBUS ****************************/

#define AMMETER_SLAVE_ADDR      73          /* ammeter slave address */

#define VOLTAGE_REG_START       0x100       /* voltage start register: 256 */

#define VOLTAGE_REG_NUM         6           /* 3 voltage values, each occupies 2 registers

*/

#define CURRENT_REG_START       0x10E       /* current start register: 270 */

#define CURRENT_REG_NUM         6           /* 3 current values, each occupies 2 registers

*/

extern float Voltage[3];        /* three-phase voltage: A, B, C */

extern float Current[3];        /* three-phase current: A, B, C */

/*************************** Ammeter MODBUS ****************************/

/*************************** Water meter MODBUS ************************/

#define WATERMETER_SLAVE_ADDR   1           /* water meter slave address */

#define FLOW_REG_START          1           /* instant flow */

#define FLOW_REG_NUM            2           /* one float value occupies 2 registers */

extern float Flow;              /* water flow */

/*************************** Water meter MODBUS ************************/

/*************************** Displacement Sensor MODBUS ****************/

#define DISPLACEMENT_SLAVE_ADDR     2           /* displacement sensor slave address (chan

ged from 1) */

#define DISPLACEMENT_REG_START      0x0000      /* displacement value register */

#define DISPLACEMENT_REG_NUM        2           /* 2 registers (only Reg[1] used) */

extern float Displacement;      /* displacement value (mm) */

/*************************** Displacement Sensor MODBUS ****************/

#define MB_POLL_CYCLE_MS   500

void send_thread_entry(void *parameter);

void mb_master_poll(void *parameter);

#endif /* APPLICATIONS_APPS_FREEMODBUSAPP_H_ */

// File: applications/Apps/hal_adc.c

/*

* Copyright (c) 2006-2021, RT-Thread Development Team

*

* SPDX-License-Identifier: Apache-2.0

*

* Change Logs:

* Date           Author       Notes

* 2025-12-05     ideapad15s       the first version

*/

#include "hal_adc.h"

ADC_HandleTypeDef hadc1;

void ADC1_Init(void)

{

hadc1.Instance = ADC1;

hadc1.Init.ClockPrescaler = ADC_CLOCK_SYNC_PCLK_DIV4;

hadc1.Init.Resolution = ADC_RESOLUTION_16B;

hadc1.Init.ScanConvMode = ADC_SCAN_DISABLE;

hadc1.Init.EOCSelection = ADC_EOC_SINGLE_CONV;

hadc1.Init.LowPowerAutoWait = DISABLE;

hadc1.Init.ContinuousConvMode = DISABLE;

hadc1.Init.NbrOfConversion = 1;

hadc1.Init.DiscontinuousConvMode = DISABLE;

hadc1.Init.ExternalTrigConv = ADC_SOFTWARE_START;

hadc1.Init.ExternalTrigConvEdge = ADC_EXTERNALTRIGCONVEDGE_NONE;

hadc1.Init.ConversionDataManagement = ADC_CONVERSIONDATA_DR;

hadc1.Init.Overrun = ADC_OVR_DATA_PRESERVED;

hadc1.Init.LeftBitShift = ADC_LEFTBITSHIFT_NONE;

hadc1.Init.OversamplingMode = DISABLE;

if (HAL_ADC_Init(&hadc1) != HAL_OK)

{

/* In RT-Thread, you can add your own error handling here if needed */

return;

}

/* ADC calibration for STM32H7 */

(void)HAL_ADCEx_Calibration_Start(&hadc1, ADC_CALIB_OFFSET, 0);

}

static uint32_t adc_read_channel(uint32_t channel)

{

uint32_t adc_value = 0;

ADC_ChannelConfTypeDef sConfig = {0};

sConfig.Channel = channel;

sConfig.Rank = ADC_REGULAR_RANK_1;

sConfig.SamplingTime = ADC_SAMPLETIME_64CYCLES_5;

sConfig.SingleDiff = ADC_SINGLE_ENDED;

sConfig.OffsetNumber = ADC_OFFSET_NONE;

sConfig.Offset = 0;

if (HAL_ADC_ConfigChannel(&hadc1, &sConfig) != HAL_OK)

{

return 0;

}

if (HAL_ADC_Start(&hadc1) == HAL_OK)

{

if (HAL_ADC_PollForConversion(&hadc1, 100) == HAL_OK)

{

adc_value = HAL_ADC_GetValue(&hadc1);

}

HAL_ADC_Stop(&hadc1);

}

return adc_value;

}

uint32_t ADC1_Read_Channel0(void)

{

return adc_read_channel(ADC_CHANNEL_0);

}

uint32_t ADC1_Read_Channel1(void)

{

return adc_read_channel(ADC_CHANNEL_15);

}

uint32_t ADC1_Read_Channel3(void)

{

return adc_read_channel(ADC_CHANNEL_3);

}

uint32_t ADC1_Read_Channel4(void)

{

return adc_read_channel(ADC_CHANNEL_19);

}

uint32_t ADC1_Read_Channel5(void)

{

return adc_read_channel(ADC_CHANNEL_5);

}

// File: applications/Apps/hal_adc.h

/*

* Copyright (c) 2006-2021, RT-Thread Development Team

*

* SPDX-License-Identifier: Apache-2.0

*

* Change Logs:

* Date           Author       Notes

* 2025-12-05     ideapad15s       the first version

*/

#ifndef APPLICATIONS_HAL_ADC_H_

#define APPLICATIONS_HAL_ADC_H_

#include "stm32h7xx_hal.h"

#include "stm32h7xx_hal_adc.h"

#include "stm32h7xx_hal_adc_ex.h"

extern ADC_HandleTypeDef hadc1;

void ADC1_Init(void);

uint32_t ADC1_Read_Channel0(void);

uint32_t ADC1_Read_Channel1(void);

uint32_t ADC1_Read_Channel3(void);

uint32_t ADC1_Read_Channel4(void);

uint32_t ADC1_Read_Channel5(void);

#endif /* APPLICATIONS_HAL_ADC_H_ */

// File: applications/Apps/huaweiCloudApp.c

﻿/*

* Copyright (c) 2006-2021, RT-Thread Development Team

*

* SPDX-License-Identifier: Apache-2.0

*

* Change Logs:

* Date           Author       Notes

* 2026-02-04     ideapad15s   Huawei Cloud IoT - ADC line sensor data upload

*/

#include "huaweiCloudApp.h"

#include "uartApp.h"

#include <string.h>

#include <stdio.h>

#define DBG_TAG "HuaweiCloud"

#define DBG_LVL (-1)

#include <rtdbg.h>

/*============================================================================

* Global ADC data - updated by linesensor.c

*============================================================================*/

rt_uint32_t g_adc_ch0 = 0;

rt_uint32_t g_adc_ch1 = 0;

rt_uint32_t g_adc_ch3 = 0;

rt_uint32_t g_adc_ch4 = 0;

rt_uint32_t g_adc_ch5 = 0;

/*============================================================================

* Private variables

*============================================================================*/

static rt_uint8_t cloud_task_step = 0;      /* current task step */

static char at_cmd_buf[256];                /* AT command buffer */

static char json_buf[600];                  /* JSON data buffer */

/*============================================================================

* Private functions

*============================================================================*/

/**

* @brief Append a float value as "key":int.dec to buf (2 decimal places)

*        RT-Thread rt_snprintf does not support %f, so we format manually.

* @return number of chars written

*/

static int append_float_field(char *buf, int buf_size, const char *key, float val)

{

int int_part = (int)val;

int dec_part = (int)((val - int_part) * 100);

if (dec_part < 0) dec_part = -dec_part;

return rt_snprintf(buf, buf_size, "\"%s\":%d.%02d", key, int_part, dec_part);

}

/**

* @brief Send AT command and wait for response

* @param cmd AT command

* @param expect Expected response string

* @param timeout Timeout in ms

* @return 1 success, 0 failure

*/

static int send_at_cmd(const char *cmd, const char *expect, rt_uint32_t timeout)

{

rt_uint32_t start_tick;

/* Clear receive buffer */

uart4_buffer_clear();

/* Send AT command */

uart4_send(cmd);

uart4_send("\r\n");

/* Wait for response */

start_tick = rt_tick_get();

while ((rt_tick_get() - start_tick) < rt_tick_from_millisecond(timeout))

{

rt_thread_mdelay(100);

if (strstr(uart4_recived_data, expect) != RT_NULL)

{

LOG_D("AT OK: %s", cmd);

return 1;

}

if (strstr(uart4_recived_data, "ERROR") != RT_NULL)

{

LOG_E("AT ERROR: %s", cmd);

return 0;

}

}

LOG_E("AT TIMEOUT: %s", cmd);

return 0;

}

/**

* @brief Wait for expected response

* @param expect Expected response string

* @param timeout Timeout in ms

* @return 1 success, 0 failure

*/

static int wait_response(const char *expect, rt_uint32_t timeout)

{

rt_uint32_t start_tick = rt_tick_get();

while ((rt_tick_get() - start_tick) < rt_tick_from_millisecond(timeout))

{

rt_thread_mdelay(100);

if (strstr(uart4_recived_data, expect) != RT_NULL)

{

return 1;

}

}

return 0;

}

/**

* @brief Build ADC data report JSON

* @note Report 5-channel ADC line sensor data

*/

static void build_adc_json(void)

{

/* Build Huawei Cloud property report JSON format */

/* {"services":[{"service_id":"STM32H743","properties":{"ch0":xxx,"ch1":xxx,...}}]} */

rt_snprintf(json_buf, sizeof(json_buf),

"{\"services\":[{\"service_id\":\"%s\",\"properties\":{\"ch0\":%u,\"ch1\":%u,\"ch3

\":%u,\"ch4\":%u,\"ch5\":%u}}]}",

SERVICE_ID_ADC,

(unsigned int)g_adc_ch0,

(unsigned int)g_adc_ch1,

(unsigned int)g_adc_ch3,

(unsigned int)g_adc_ch4,

(unsigned int)g_adc_ch5);

}

/*============================================================================

* Other sensor report functions (commented out, uncomment when needed)

*============================================================================*/

#if 0  /* ===== Other sensor report code - currently disabled ===== */

/**

* @brief Build Node data report JSON (Modbus collected data)

* @note Includes: Flow, Flame, Methane, 3-phase current

* @note Before use:

*       1. Create corresponding service and properties on Huawei Cloud

*       2. Modify SERVICE_ID to match

*       3. Ensure node[] array has valid data

*/

/*

static void build_node_json(void)

{

extern Node node[NODENUM];  // Reference node data from NodeApp.h

// Report node[0] data

rt_snprintf(json_buf, sizeof(json_buf),

"{\"services\":[{\"service_id\":\"Node_1\",\"properties\":{"

"\"Flow\":%.2f,"

"\"Flame\":%d,"

"\"Methane\":%.2f,"

"\"current_A\":%.2f,"

"\"current_B\":%.2f,"

"\"current_C\":%.2f"

"}}]}",

node[0].Flow,

(int)node[0].Flame,

node[0].Methane,

node[0].CH1_A[0],

node[0].CH1_A[1],

node[0].CH1_A[2]);

}

*/

/**

* @brief Build methane sensor data report JSON

* @note Before use:

*       1. Create methane sensor service on Huawei Cloud

*       2. Add properties: methane_ppm(int), methane_lel(int)

*/

/*

static void build_methane_json(rt_uint16_t ppm, rt_uint8_t lel)

{

rt_snprintf(json_buf, sizeof(json_buf),

"{\"services\":[{\"service_id\":\"MethaneSensor\",\"properties\":{"

"\"methane_ppm\":%u,"

"\"methane_lel\":%u"