* Format:

* Flame: xx,xx,xx,xx,xx

* gas: ppm: xx , level: xx%

* voltage_a: xxV,voltage_b: xxV,voltage_c: xxV

* current_a: xxA,current_b: xxA,current_c: xxA

* flow: xxm³/h

*/

static void lora_build_text(char *buf, rt_size_t size)

{

int voltage_a_int = (int)Voltage[0];

int voltage_b_int = (int)Voltage[1];

int voltage_c_int = (int)Voltage[2];

int current_a_int = (int)Current[0];

int current_b_int = (int)Current[1];

int current_c_int = (int)Current[2];

int flow_int = (int)Flow;

rt_snprintf(buf, size,

"Flame: %u,%u,%u,%u,%u\r\n"

"gas: ppm: %u , level: %u%%\r\n"

"voltage_a: %dV,voltage_b: %dV,voltage_c: %dV\r\n"

"current_a: %dA,current_b: %dA,current_c: %dA\r\n"

"flow: %d m^3/h\r\n",

(unsigned int)g_adc_ch0,

(unsigned int)g_adc_ch1,

(unsigned int)g_adc_ch3,

(unsigned int)g_adc_ch4,

(unsigned int)g_adc_ch5,

(unsigned int)g_methane_ppm,

(unsigned int)g_methane_lel,

voltage_a_int,

voltage_b_int,

voltage_c_int,

current_a_int,

current_b_int,

current_c_int,

flow_int);

}

/**

* @brief LoRa send thread entry

*

* Initializes UART5, then periodically sends all sensor data

* as text via LoRa transparent mode.

*/

void lora_thread_entry(void *parameter)

{

char text_buf[LORA_TEXT_BUF_SIZE];

uart5_init();

LOG_I("LoRa thread started (UART5, 115200 baud).");

rt_thread_mdelay(3000);

while (1)

{

lora_build_text(text_buf, sizeof(text_buf));

uart5_send(text_buf);

rt_thread_mdelay(LORA_SEND_INTERVAL);

}

}

// File: applications/Apps/loraApp.h

/*

* Copyright (c) 2006-2021, RT-Thread Development Team

*

* SPDX-License-Identifier: Apache-2.0

*

* Change Logs:

* Date           Author       Notes

* 2026-04-13     ideapad15s   ATK-LORA-01 module via UART5

*/

#ifndef APPLICATIONS_APPS_LORAAPP_H_

#define APPLICATIONS_APPS_LORAAPP_H_

#include <rtthread.h>

#define LORA_SEND_INTERVAL      5000    /* send interval (ms) */

void lora_thread_entry(void *parameter);

#endif /* APPLICATIONS_APPS_LORAAPP_H_ */

// File: applications/Apps/MethaneSensorApp.c

﻿/*

* Copyright (c) 2006-2021, RT-Thread Development Team

*

* SPDX-License-Identifier: Apache-2.0

*

* Change Logs:

* Date           Author       Notes

* 2025-11-17     ideapad15s       the first version

*/

#include "heads.h"

#include <drivers/serial.h>

#include "MethaneSensorApp.h"

#define UART2_DEVICE_NAME    "uart2" /* UART2_TX---PD5   UART2_RX---PD6 (NOT PA2/PA3!) */

#define PACKET_SIZE          19      /* fixed 19-byte packet */

/* IMPORTANT: Hardware connection for GM-402B methane sensor:

* - Sensor TX  -> MCU PD6 (UART2_RX)

* - Sensor RX  -> MCU PD5 (UART2_TX)

* - Sensor GND -> MCU GND

* - Sensor VIN -> 3.3V or 5V (check sensor spec)

*

* NOTE: PA2 and PA3 are NOT used for UART2!

*       PA2 is used by ETH_MDIO (Ethernet)

*       PA3 is used by ADC1_INP15 (Flame sensor A2)

*/

/* global methane sensor data, for cloud report */

rt_uint16_t g_methane_ppm = 0;

rt_uint8_t  g_methane_lel = 0;

rt_uint8_t  g_methane_status = 0;

rt_bool_t   g_methane_data_valid = RT_FALSE;

static rt_uint32_t methane_valid_frames = 0;

static rt_uint32_t methane_invalid_frames = 0;

static volatile rt_bool_t methane_debug_active = RT_FALSE;

/* receive packet buffer */

static rt_uint8_t uart2_received_data[PACKET_SIZE];

/**

* @brief Validate and parse one complete methane sensor frame

*

* @param buffer_in input packet buffer, fixed 19 bytes

*/

static rt_bool_t Response_FrameCheck_Uart(const rt_uint8_t *buffer_in)

{

rt_uint8_t checksum = 0;

rt_uint16_t methane = 0;

rt_uint8_t lel = 0;

if (buffer_in[0] != 0xAC || buffer_in[1] != 0xAC ||

buffer_in[2] != PACKET_SIZE || buffer_in[3] != 0xAA)

{

methane_invalid_frames++;

return RT_FALSE;

}

/* checksum verification */

for (rt_uint8_t i = 0; i < PACKET_SIZE - 1; i++)

{

checksum += buffer_in[i];

}

if (checksum != buffer_in[PACKET_SIZE - 1])

{

methane_invalid_frames++;

return RT_FALSE;

}

/* Manufacturer reference implementation (Ver1.1):

* [10] = PPM low byte, [11] = PPM high byte (little-endian)

* [12] = LEL %, [13] = device/fault status.

* Bytes [8] and [9] are not the PPM field. */

methane = ((rt_uint16_t)buffer_in[11] << 8) | (rt_uint16_t)buffer_in[10];

lel = buffer_in[12];

g_methane_ppm = methane;

g_methane_lel = lel;

g_methane_status = buffer_in[13];

g_methane_data_valid = RT_TRUE;

methane_valid_frames++;

return RT_TRUE;

}

/**

* @brief Receive UART2 data and print in hexadecimal format

*

* @param timeout receive timeout in ms, -1 means wait forever

* @return rt_err_t RT_EOK on success, negative value on failure

*/

rt_err_t uart2_receive_and_print(rt_int32_t timeout)

{

rt_device_t serial;

rt_uint8_t data;

rt_size_t result;

rt_uint8_t data_index = 0; /* number of bytes collected in current frame */

/* find UART device */

serial = rt_device_find(UART2_DEVICE_NAME);

if (serial == RT_NULL)

{

/* rt_kprintf("Cannot find %s device!\n", UART2_DEVICE_NAME); */

return -RT_ERROR;

}

/* open device in read/write mode */

if (rt_device_open(serial, RT_DEVICE_FLAG_RDWR | RT_DEVICE_FLAG_INT_RX) != RT_EOK)

{

/* rt_kprintf("Failed to open %s device!\n", UART2_DEVICE_NAME); */

return -RT_ERROR;

}

/* configure UART baud rate to 19200 */

struct serial_configure config = RT_SERIAL_CONFIG_DEFAULT;

config.baud_rate = BAUD_RATE_19200;

if (rt_device_control(serial, RT_DEVICE_CTRL_CONFIG, &config) != RT_EOK)

{

/* rt_kprintf("Failed to configure UART2 baud rate!\n"); */

rt_device_close(serial);

return -RT_ERROR;

}

/* rt_kprintf("UART2 receive started (19200 baud), waiting for data...\n"); */

/* rt_kprintf("Received HEX data:\n"); */

/* receive loop */

while (1)

{

if (methane_debug_active)

{

data_index = 0;

rt_thread_mdelay(10);

continue;

}

/* read one byte from UART */

result = rt_device_read(serial, 0, &data, 1);

if (result > 0)

{

/* Synchronize on AC AC, then collect exactly one 19-byte frame. */

if (data_index == 0)

{

if (data == 0xAC)

{

uart2_received_data[0] = data;

data_index = 1;

}

}

else if (data_index == 1)

{

if (data == 0xAC)

{

uart2_received_data[1] = data;

data_index = 2;

}

else

{

data_index = 0;

}

}

else

{

uart2_received_data[data_index++] = data;

if (data_index == PACKET_SIZE)

{

(void)Response_FrameCheck_Uart(uart2_received_data);

data_index = 0;

}

}

}

else if (timeout > 0)

{

/* timeout mode: wait a little and decrease timeout */

rt_thread_mdelay(10);

timeout -= 10;

if (timeout <= 0)

{

/* rt_kprintf("\nUART2 receive timeout!\n"); */

break;

}

}

else if (timeout == 0)

{

/* timeout is zero, exit directly */

break;

}

else

{

/* timeout < 0 means wait forever */

rt_thread_mdelay(10);

}

}

/* close device */

rt_device_close(serial);

return RT_EOK;

}

/**

* @brief MSH command: debug UART2 raw data reception

*

* Usage: uart2_debug [seconds]

* Example: uart2_debug 10  (monitor for 10 seconds)

*/

static void uart2_debug(int argc, char **argv)

{

rt_device_t serial;

rt_uint8_t data;

rt_size_t result;

rt_uint32_t byte_count = 0;

rt_uint32_t debug_valid_before;

rt_uint32_t debug_invalid_before;

rt_uint32_t timeout_ms = 5000;  /* default 5 seconds */

rt_tick_t start_tick;

rt_uint8_t frame[PACKET_SIZE];

rt_uint8_t frame_index = 0;

if (argc == 2)

{

int seconds = atoi(argv[1]);

if (seconds > 0 && seconds <= 60)

{

timeout_ms = seconds * 1000;

}

}

methane_debug_active = RT_TRUE;

rt_thread_mdelay(20);

rt_kprintf("\n========================================\n");

rt_kprintf("  UART2 Raw Data Debug Monitor\n");

rt_kprintf("========================================\n");

rt_kprintf("Device: %s\n", UART2_DEVICE_NAME);

rt_kprintf("Pins: PD5 (TX), PD6 (RX)\n");

rt_kprintf("Baud: 19200\n");

rt_kprintf("Duration: %d seconds\n", timeout_ms / 1000);

rt_kprintf("Expected frame: AC AC 13 AA ...(19 bytes)\n");

rt_kprintf("========================================\n\n");

/* find UART device */

serial = rt_device_find(UART2_DEVICE_NAME);

if (serial == RT_NULL)

{

rt_kprintf("ERROR: Cannot find %s device!\n", UART2_DEVICE_NAME);

methane_debug_active = RT_FALSE;

return;

}

/* open device */

if (rt_device_open(serial, RT_DEVICE_FLAG_RDWR | RT_DEVICE_FLAG_INT_RX) != RT_EOK)

{

rt_kprintf("ERROR: Failed to open %s device!\n", UART2_DEVICE_NAME);

methane_debug_active = RT_FALSE;

return;

}

/* configure UART baud rate to 19200 for GM-402B methane sensor */

struct serial_configure config = RT_SERIAL_CONFIG_DEFAULT;

config.baud_rate = BAUD_RATE_19200;

if (rt_device_control(serial, RT_DEVICE_CTRL_CONFIG, &config) != RT_EOK)

{

rt_kprintf("ERROR: Failed to configure UART2 baud rate!\n");

rt_device_close(serial);

methane_debug_active = RT_FALSE;

return;

}

rt_kprintf("Receiving data (HEX format):\n");

/* Temporarily stop the background parser.  Both readers consume the same

* RT-Thread UART RX queue; allowing them to run together loses bytes and

* makes a valid stream appear to have a non-19-byte length. */

debug_valid_before = methane_valid_frames;

debug_invalid_before = methane_invalid_frames;

start_tick = rt_tick_get();

/* receive loop with timeout */

while (1)

{

/* check timeout */

if ((rt_tick_get() - start_tick) >= rt_tick_from_millisecond(timeout_ms))

{

break;

}

/* read one byte from UART */

result = rt_device_read(serial, 0, &data, 1);

if (result > 0)

{

/* print byte in hex */

rt_kprintf("%02X ", data);

byte_count++;

/* Independently detect and validate complete frames in the raw

* stream, rather than judging validity by total byte count. */

if (frame_index == 0)

{

if (data == 0xAC)

{

frame[0] = data;

frame_index = 1;

}

}

else if (frame_index == 1)

{

if (data == 0xAC)

{

frame[1] = data;

frame_index = 2;

}

else

{

frame_index = 0;

}

}

else

{

frame[frame_index++] = data;

if (frame_index == PACKET_SIZE)

{

(void)Response_FrameCheck_Uart(frame);

frame_index = 0;

rt_kprintf("\n");

}

}

if ((byte_count % PACKET_SIZE == 0) && (frame_index != 0))

{

rt_kprintf("\n");

}

}

else

{

/* no data, wait a bit */

rt_thread_mdelay(10);

}

}

rt_device_close(serial);

methane_debug_active = RT_FALSE;

rt_kprintf("\n\n========================================\n");

rt_kprintf("Total bytes received: %d\n", byte_count);

if (byte_count == 0)

{

rt_kprintf("\nWARNING: No data received!\n");

rt_kprintf("Possible causes:\n");

rt_kprintf("  1. Sensor not powered (check VIN and GND)\n");

rt_kprintf("  2. Wrong pin connection - should be:\n");

rt_kprintf("     Sensor TX -> MCU PD6 (UART2_RX)\n");

rt_kprintf("     Sensor RX -> MCU PD5 (UART2_TX)\n");

rt_kprintf("  3. Sensor requires warm-up time (wait 2-3 minutes)\n");

rt_kprintf("  4. Check if sensor is actually GM-402B module\n");

}

else

{

rt_uint32_t valid_count = methane_valid_frames - debug_valid_before;

rt_kprintf("\nValid frames: %u\n", (unsigned int)valid_count);

rt_kprintf("Invalid frames: %u\n",

(unsigned int)(methane_invalid_frames - debug_invalid_before));

if (frame_index != 0)

rt_kprintf("Incomplete trailing bytes: %d\n", frame_index);

if (valid_count == 0)

rt_kprintf("WARNING: Data received, but no valid GM-402B frame found.\n");

else

rt_kprintf("Last value: %u ppm, LEL %u%%, status 0x%02X\n",

g_methane_ppm, g_methane_lel, g_methane_status);

}

rt_kprintf("========================================\n\n");

}

MSH_CMD_EXPORT_ALIAS(uart2_debug, uart2_debug, Debug UART2 raw data (uart2_debug [seconds]

));

// File: applications/Apps/MethaneSensorApp.h

/*

* Copyright (c) 2006-2021, RT-Thread Development Team

*

* SPDX-License-Identifier: Apache-2.0

*

* Change Logs:

* Date           Author       Notes

* 2025-02-28     Administrator       the first version

*/

#ifndef APPLICATIONS_APPS_METHANESENSORAPP_H_

#define APPLICATIONS_APPS_METHANESENSORAPP_H_

#include "heads.h"

extern float Methane;           /* methane concentration */

extern rt_uint16_t g_methane_ppm;   /* methane ppm for cloud report */

extern rt_uint8_t  g_methane_lel;   /* methane LEL% for cloud report */

extern rt_uint8_t  g_methane_status;/* GM-402B status byte ([13], 0xAA=normal) */

extern rt_bool_t   g_methane_data_valid; /* at least one valid frame received */

rt_err_t uart2_receive_and_print(rt_int32_t timeout);

#endif /* APPLICATIONS_APPS_METHANESENSORAPP_H_ */

// File: applications/Apps/NodeApp.c

/*

* Copyright (c) 2006-2021, RT-Thread Development Team

*

* SPDX-License-Identifier: Apache-2.0

*

* Change Logs:

* Date           Author       Notes

* 2025-03-01     Administrator       the first version

*/

#include "NodeApp.h"

Node node[NODENUM] = {0};

// File: applications/Apps/NodeApp.h

/*

* Copyright (c) 2006-2021, RT-Thread Development Team

*

* SPDX-License-Identifier: Apache-2.0

*

* Change Logs:

* Date           Author       Notes

* 2025-03-01     Administrator       the first version

*/

#ifndef APPLICATIONS_APPS_NODEAPP_H_

#define APPLICATIONS_APPS_NODEAPP_H_

typedef struct Node

{

float CH1_A[3];             //0为AC相电流，1为BC相电流，2为CA相电流

float Flow;                 //流量

char Flame;                 //火焰传感器 1有火 0无火

float Methane;              //甲烷浓度

float Displacement;         //位移值 (mm)

} Node;

#define NODENUM 2

extern Node node[NODENUM];

#endif /* APPLICATIONS_APPS_NODEAPP_H_ */

// File: applications/Apps/o2SensorApp.c

/*

* Copyright (c) 2006-2026, RT-Thread Development Team

*

* SPDX-License-Identifier: Apache-2.0

*

* Change Logs:

* Date           Author       Notes

* 2026-04-22     ideapad15s   O2 sensor via ADC1

* 2026-07-11     ideapad15s   use ADC1 channel 18 (PA4)

*/

#include "heads.h"

#include "o2SensorApp.h"

#define DBG_TAG "O2"

#define DBG_LVL DBG_ERROR

#include <rtdbg.h>

/* Global sensor data */

float g_o2_concentration = 0.0f;

static rt_adc_device_t adc_dev = RT_NULL;

/**

* @brief Read ADC raw value, convert to O2 concentration %.

* @param o2_percent [out] O2 concentration in %

* @return RT_EOK on success, negative on error

*/

static rt_err_t o2_read_once(float *o2_percent)

{

rt_uint32_t raw;

rt_int32_t  o2_x10;

raw = rt_adc_read(adc_dev, O2_ADC_CHANNEL);

/* Direct ADC to O2% conversion (reference code uses ADC raw value, not mV!)

* Formula: O2%(×10) = (ADC_raw - zero) * 209 / (full - zero)

* Where full = 2410 is the ADC raw value at 20.9% O2 (air)

*/

o2_x10 = (rt_int32_t)(raw - O2_ZERO_OFFSET) * 209

/ (rt_int32_t)(O2_FULL_SCALE_ADC - O2_ZERO_OFFSET);

/* Air-stabilize: near 20.9% → force to 20.9% */

if (o2_x10 >= O2_AIR_STABILIZE_LOW && o2_x10 <= O2_AIR_STABILIZE_HIGH)

{

o2_x10 = O2_AIR_STABILIZE_VALUE;

}

*o2_percent = (float)o2_x10 / 10.0f;

return RT_EOK;

}

/**

* @brief O2 sensor acquisition thread entry

*/

void o2_thread_entry(void *parameter)

{

float o2;

LOG_I("O2 thread started on %s ch%d", O2_ADC_DEVICE_NAME, O2_ADC_CHANNEL);

while (1)

{

if (o2_read_once(&o2) == RT_EOK)

{

g_o2_concentration = o2;

}

rt_thread_mdelay(O2_READ_INTERVAL_MS);

}

}

/**

* @brief Initialize O2 sensor: find ADC device, create and start thread

*/

rt_err_t o2_sensor_init(void)

{

adc_dev = (rt_adc_device_t)rt_device_find(O2_ADC_DEVICE_NAME);

if (adc_dev == RT_NULL)

{

LOG_E("ADC device %s not found", O2_ADC_DEVICE_NAME);

return -RT_ERROR;

}

if (rt_adc_enable(adc_dev, O2_ADC_CHANNEL) != RT_EOK)

{

LOG_E("ADC enable ch%d failed", O2_ADC_CHANNEL);

return -RT_ERROR;

}

rt_thread_t tid = rt_thread_create("o2_rd",

o2_thread_entry,

RT_NULL,

1024,

25,

10);

if (tid == RT_NULL)

{

LOG_E("Failed to create O2 thread");

return -RT_ERROR;

}

rt_thread_startup(tid);

LOG_I("O2 sensor initialized on %s ch%d", O2_ADC_DEVICE_NAME, O2_ADC_CHANNEL);

return RT_EOK;

}

// File: applications/Apps/o2SensorApp.h

/*

* Copyright (c) 2006-2026, RT-Thread Development Team

*

* SPDX-License-Identifier: Apache-2.0

*

* Change Logs:

* Date           Author       Notes

* 2026-04-22     ideapad15s   O2 sensor via ADC1 channel 8 (PC5)

* 2026-06-18     ideapad15s   migrate from PC5 for ETH RMII

* 2026-07-11     ideapad15s   use ADC1 channel 18 (PA4), exposed on core-board header

*/

#ifndef APPLICATIONS_APPS_O2SENSORAPP_H_

#define APPLICATIONS_APPS_O2SENSORAPP_H_

#include <rtthread.h>

/* ADC config */

#define O2_ADC_DEVICE_NAME          "adc1"

#define O2_ADC_CHANNEL              18

/* Read interval (ms) — same as SHT30 */

#define O2_READ_INTERVAL_MS         2000

/* Calibration — adjust per sensor unit

* These are ADC raw values (0-65535), NOT millivolts!

* Full scale 2410 means: at 20.9% O2 (air), ADC reads ~2410

*/

#define O2_ZERO_OFFSET              0

#define O2_FULL_SCALE_ADC           2410

/* Air-stabilize zone (×10 values) */

#define O2_AIR_STABILIZE_LOW        207

#define O2_AIR_STABILIZE_HIGH       211

#define O2_AIR_STABILIZE_VALUE      209

/* Global sensor data — O2 concentration in % */

extern float g_o2_concentration;

/* Thread entry */

void o2_thread_entry(void *parameter);

/* Init: create and start the O2 acquisition thread */

rt_err_t o2_sensor_init(void);

#endif /* APPLICATIONS_APPS_O2SENSORAPP_H_ */

// File: applications/Apps/sensor_debug_config.h

/*

* Copyright (c) 2006-2026, RT-Thread Development Team

*

* SPDX-License-Identifier: Apache-2.0

*

* Change Logs:

* Date           Author       Notes

* 2026-09-18     ideapad15s   Sensor debug configuration

*/

#ifndef APPLICATIONS_APPS_SENSOR_DEBUG_CONFIG_H_

#define APPLICATIONS_APPS_SENSOR_DEBUG_CONFIG_H_

/**

* 传感器调试开关

*

* 在逐个调试传感器时，可以关闭其他传感器以减少干扰

* 调试顺序：

* 1. 火焰传感器

* 2. 温湿度传感器（SHT30）

* 3. 甲烷气体传感器

* 4. 氧气传感器

* 5. 位移传感器

* 6. 电表（三相）

*/

/* ============================================

* 调试阶段选择（取消注释对应阶段）

* ============================================ */

/* 阶段1: 火焰传感器调试 */

// #define DEBUG_STAGE_1_FLAME

/* 阶段2: 温湿度传感器调试 */

// #define DEBUG_STAGE_2_SHT30

/* 阶段3: 甲烷传感器调试 */

// #define DEBUG_STAGE_3_METHANE

/* 阶段4: 氧气传感器调试 */

// #define DEBUG_STAGE_4_O2

/* 阶段5: 位移传感器调试 */

// #define DEBUG_STAGE_5_DISPLACEMENT

/* 阶段7: 电表调试 */

// #define DEBUG_STAGE_7_AMMETER

/* 全功能模式：所有传感器同时运行（生产环境） */

#define DEBUG_STAGE_ALL_SENSORS

/* ============================================

* 根据调试阶段自动配置传感器使能

* ============================================ */

#if defined(DEBUG_STAGE_1_FLAME)

/* 阶段1：仅启用火焰传感器 */

#define ENABLE_FLAME_SENSOR         1

#define ENABLE_SHT30_SENSOR         0

#define ENABLE_METHANE_SENSOR       0

#define ENABLE_O2_SENSOR            0

#define ENABLE_DISPLACEMENT_SENSOR  0

#define ENABLE_STRESS_SENSOR        0

#define ENABLE_AMMETER              0

#define ENABLE_WIFI_CLOUD           0

#define ENABLE_LORA                 0

#define ENABLE_ETHERNET             0

#elif defined(DEBUG_STAGE_2_SHT30)

/* 阶段2：启用火焰+温湿度 */

#define ENABLE_FLAME_SENSOR         1

#define ENABLE_SHT30_SENSOR         1

#define ENABLE_METHANE_SENSOR       0

#define ENABLE_O2_SENSOR            0

#define ENABLE_DISPLACEMENT_SENSOR  0

#define ENABLE_STRESS_SENSOR        0

#define ENABLE_AMMETER              0

#define ENABLE_WIFI_CLOUD           0

#define ENABLE_LORA                 0

#define ENABLE_ETHERNET             0

#elif defined(DEBUG_STAGE_3_METHANE)

/* 阶段3：启用火焰+温湿度+甲烷 */

#define ENABLE_FLAME_SENSOR         1

#define ENABLE_SHT30_SENSOR         1

#define ENABLE_METHANE_SENSOR       1

#define ENABLE_O2_SENSOR            0

#define ENABLE_DISPLACEMENT_SENSOR  0

#define ENABLE_STRESS_SENSOR        0

#define ENABLE_AMMETER              0

#define ENABLE_WIFI_CLOUD           0

#define ENABLE_LORA                 0

#define ENABLE_ETHERNET             0

#elif defined(DEBUG_STAGE_4_O2)

/* 阶段4：启用火焰+温湿度+甲烷+氧气 */

#define ENABLE_FLAME_SENSOR         1

#define ENABLE_SHT30_SENSOR         1

#define ENABLE_METHANE_SENSOR       1

#define ENABLE_O2_SENSOR            1

#define ENABLE_DISPLACEMENT_SENSOR  0

#define ENABLE_STRESS_SENSOR        0

#define ENABLE_AMMETER              0

#define ENABLE_WIFI_CLOUD           0

#define ENABLE_LORA                 0

#define ENABLE_ETHERNET             0

#elif defined(DEBUG_STAGE_5_DISPLACEMENT)

/* 阶段5：启用所有前置传感器+位移传感器 */

#define ENABLE_FLAME_SENSOR         1

#define ENABLE_SHT30_SENSOR         1

#define ENABLE_METHANE_SENSOR       1

#define ENABLE_O2_SENSOR            1

#define ENABLE_DISPLACEMENT_SENSOR  1

#define ENABLE_STRESS_SENSOR        0

#define ENABLE_AMMETER              0

#define ENABLE_WIFI_CLOUD           0

#define ENABLE_LORA                 0

#define ENABLE_ETHERNET             0

#elif defined(DEBUG_STAGE_7_AMMETER)

/* 阶段7：启用所有传感器+电表 */

#define ENABLE_FLAME_SENSOR         1

#define ENABLE_SHT30_SENSOR         1

#define ENABLE_METHANE_SENSOR       1

#define ENABLE_O2_SENSOR            1

#define ENABLE_DISPLACEMENT_SENSOR  1

#define ENABLE_STRESS_SENSOR        1

#define ENABLE_AMMETER              1

#define ENABLE_WIFI_CLOUD           0

#define ENABLE_LORA                 0

#define ENABLE_ETHERNET             0

#elif defined(DEBUG_STAGE_ALL_SENSORS)

/* 全功能模式：所有功能启用（生产环境） */

#define ENABLE_FLAME_SENSOR         1

#define ENABLE_SHT30_SENSOR         1

#define ENABLE_METHANE_SENSOR       1

#define ENABLE_O2_SENSOR            1

#define ENABLE_DISPLACEMENT_SENSOR  1

#define ENABLE_STRESS_SENSOR        1

#define ENABLE_AMMETER              1

#define ENABLE_WIFI_CLOUD           1

#define ENABLE_LORA                 1

#define ENABLE_ETHERNET             1

#else

/* 默认：启用所有传感器 */

#define ENABLE_FLAME_SENSOR         1

#define ENABLE_SHT30_SENSOR         1

#define ENABLE_METHANE_SENSOR       1

#define ENABLE_O2_SENSOR            1

#define ENABLE_DISPLACEMENT_SENSOR  1

#define ENABLE_STRESS_SENSOR        1

#define ENABLE_AMMETER              1

#define ENABLE_WIFI_CLOUD           1

#define ENABLE_LORA                 1

#define ENABLE_ETHERNET             1

#endif

/* ============================================

* 调试输出开关

* ============================================ */

/* 启用调试输出（会在串口打印详细的传感器数据） */

#define DEBUG_PRINT_FLAME           0   /* 火焰传感器调试输出 */

#define DEBUG_PRINT_SHT30           0   /* SHT30调试输出 */

#define DEBUG_PRINT_METHANE         0   /* 甲烷传感器调试输出 */

#define DEBUG_PRINT_O2              0   /* 氧气传感器调试输出 */

#define DEBUG_PRINT_DISPLACEMENT    0   /* 位移传感器调试输出 */

#define DEBUG_PRINT_AMMETER         0   /* 电表调试输出 */

#define DEBUG_PRINT_MODBUS          0   /* Modbus通信调试输出 */

/* ============================================

* 调试辅助宏

* ============================================ */

/* 根据调试阶段打印提示信息 */

#if defined(DEBUG_STAGE_1_FLAME)

#define DEBUG_STAGE_NAME "Stage 1: Flame Sensor Debug"

#elif defined(DEBUG_STAGE_2_SHT30)

#define DEBUG_STAGE_NAME "Stage 2: SHT30 Sensor Debug"

#elif defined(DEBUG_STAGE_3_METHANE)

#define DEBUG_STAGE_NAME "Stage 3: Methane Sensor Debug"

#elif defined(DEBUG_STAGE_4_O2)

#define DEBUG_STAGE_NAME "Stage 4: O2 Sensor Debug"

#elif defined(DEBUG_STAGE_5_DISPLACEMENT)

#define DEBUG_STAGE_NAME "Stage 5: Displacement Sensor Debug"

#elif defined(DEBUG_STAGE_7_AMMETER)

#define DEBUG_STAGE_NAME "Stage 7: Ammeter Debug"

#elif defined(DEBUG_STAGE_ALL_SENSORS)

#define DEBUG_STAGE_NAME "All Sensors Enabled (Production Mode)"

#else

#define DEBUG_STAGE_NAME "Default Mode"

#endif

/* 打印当前调试阶段 */

#define PRINT_DEBUG_STAGE() \

rt_kprintf("\n========================================\n"); \

rt_kprintf("  %s\n", DEBUG_STAGE_NAME); \

rt_kprintf("========================================\n\n")

/* ============================================

* 传感器采样周期配置（单位：ms）

* ============================================ */

#define FLAME_SAMPLE_PERIOD         500     /* 火焰传感器采样周期 */

#define SHT30_SAMPLE_PERIOD         2000    /* SHT30采样周期 */

#define METHANE_SAMPLE_PERIOD       1000    /* 甲烷传感器采样周期 */

#define O2_SAMPLE_PERIOD            1000    /* 氧气传感器采样周期 */

#define DISPLACEMENT_SAMPLE_PERIOD  500     /* 位移传感器采样周期 */

#define AMMETER_SAMPLE_PERIOD       500     /* 电表采样周期 */

/* ============================================

* Modbus总线配置

* ============================================ */

/* Modbus设备优先级（值越小优先级越高） */

#define MODBUS_PRIORITY_DISPLACEMENT  1   /* 位移传感器优先级最高 */

#define MODBUS_PRIORITY_AMMETER       2   /* 电表次之 */

/* Modbus超时配置（单位：ms） */

#define MODBUS_TIMEOUT_DISPLACEMENT   1000  /* 位移传感器超时 */

#define MODBUS_TIMEOUT_AMMETER        1000  /* 电表超时 */

#endif /* APPLICATIONS_APPS_SENSOR_DEBUG_CONFIG_H_ */

// File: applications/Apps/sht30App.c

﻿/*

* Copyright (c) 2006-2026, RT-Thread Development Team

*

* SPDX-License-Identifier: Apache-2.0

*

* Change Logs:

* Date           Author       Notes

* 2026-04-22     ideapad15s   SHT30 temperature/humidity sensor via I2C1

*/

#include "heads.h"

#include "sht30App.h"

#define DBG_TAG "SHT30"

#define DBG_LVL (-1)

#include <rtdbg.h>

/* SHT30 CRC8: polynomial 0x31, init 0xFF */

static uint8_t sht30_crc8(const uint8_t *data, int len)

{

uint8_t crc = 0xFF;

int i, j;

for (i = 0; i < len; i++)

{

crc ^= data[i];

for (j = 0; j < 8; j++)

{

if (crc & 0x80)

crc = (uint8_t)((crc << 1) ^ 0x31);

else

crc <<= 1;

}

}

return crc;

}

/* Global sensor data */

float g_temperature_c = 0.0f;

float g_humidity_rh    = 0.0f;

static struct rt_i2c_bus_device *i2c_bus = RT_NULL;

/**

* @brief Send measurement command and read 6 bytes from SHT30.

* @param temp_c  [out] parsed temperature in Celsius

* @param humi_rh [out] parsed relative humidity in %

* @return RT_EOK on success, negative on error

*/

static rt_err_t sht30_read_once(float *temp_c, float *humi_rh)

{

uint8_t cmd[2] = SHT30_CMD_MEAS_HIGH_REP;

uint8_t buf[6];

struct rt_i2c_msg msgs[1];

rt_uint16_t raw_t, raw_h;

rt_err_t ret;

/* Send measurement command */

msgs[0].addr  = SHT30_I2C_ADDR;

msgs[0].flags = RT_I2C_WR;

msgs[0].buf   = cmd;

msgs[0].len   = 2;

ret = rt_i2c_transfer(i2c_bus, msgs, 1);

if (ret != 1)

{

LOG_E("I2C write cmd failed, ret=%d", ret);

return -RT_ERROR;

}

/* Wait for measurement to complete (>15ms for high repeatability) */

rt_thread_mdelay(20);

/* Read 6 bytes: temp MSB, temp LSB, temp CRC, humi MSB, humi LSB, humi CRC */

msgs[0].addr  = SHT30_I2C_ADDR;

msgs[0].flags = RT_I2C_RD;

msgs[0].buf   = buf;

msgs[0].len   = 6;

ret = rt_i2c_transfer(i2c_bus, msgs, 1);

if (ret != 1)

{

LOG_E("I2C read data failed, ret=%d", ret);

return -RT_ERROR;

}

/* CRC8 verification on temperature bytes (buf[0..1] vs buf[2]) */

if (sht30_crc8(buf, 2) != buf[2])

{

LOG_E("CRC8 fail on temperature bytes: %02X %02X %02X",

buf[0], buf[1], buf[2]);

return -RT_ERROR;

}

/* CRC8 verification on humidity bytes (buf[3..4] vs buf[5]) */

if (sht30_crc8(&buf[3], 2) != buf[5])

{

LOG_E("CRC8 fail on humidity bytes: %02X %02X %02X",

buf[3], buf[4], buf[5]);

return -RT_ERROR;

}

raw_t = ((rt_uint16_t)buf[0] << 8) | buf[1];

raw_h = ((rt_uint16_t)buf[3] << 8) | buf[4];

*temp_c  = -45.0f + 175.0f * (float)raw_t / 65535.0f;

*humi_rh = 100.0f * (float)raw_h / 65535.0f;

return RT_EOK;

}

/**

* @brief SHT30 acquisition thread entry

*/

void sht30_thread_entry(void *parameter)

{

float temp, humi;

LOG_I("SHT30 thread started on %s", SHT30_I2C_BUS_NAME);

while (1)

{

if (sht30_read_once(&temp, &humi) == RT_EOK)

{

g_temperature_c = temp;

g_humidity_rh    = humi;

}

rt_thread_mdelay(SHT30_READ_INTERVAL_MS);

}

}

/**

* @brief Initialize SHT30: find I2C bus, create and start thread

*/

rt_err_t sht30_init(void)

{

i2c_bus = rt_i2c_bus_device_find(SHT30_I2C_BUS_NAME);

if (i2c_bus == RT_NULL)

{

LOG_E("I2C bus %s not found", SHT30_I2C_BUS_NAME);

return -RT_ERROR;

}

rt_thread_t tid = rt_thread_create("sht30_rd",

sht30_thread_entry,

RT_NULL,

1024,

25,

10);

if (tid == RT_NULL)

{

LOG_E("Failed to create SHT30 thread");

return -RT_ERROR;

}

rt_thread_startup(tid);

LOG_I("SHT30 initialized on %s", SHT30_I2C_BUS_NAME);

return RT_EOK;

}

// File: applications/Apps/sht30App.h

/*

* Copyright (c) 2006-2026, RT-Thread Development Team

*

* SPDX-License-Identifier: Apache-2.0

*

* Change Logs:

* Date           Author       Notes

* 2026-04-22     ideapad15s   SHT30 temperature/humidity sensor via I2C1

*/

#ifndef APPLICATIONS_APPS_SHT30APP_H_

#define APPLICATIONS_APPS_SHT30APP_H_

#include <rtthread.h>

/* I2C config */

#define SHT30_I2C_BUS_NAME          "i2c1"

#define SHT30_I2C_ADDR              0x44

/* Measurement command: single-shot, high repeatability */

#define SHT30_CMD_MEAS_HIGH_REP     {0x2C, 0x06}

/* Read interval (ms) */

#define SHT30_READ_INTERVAL_MS      2000

/* Global sensor data */

extern float g_temperature_c;   /* Temperature in Celsius */

extern float g_humidity_rh;     /* Relative humidity in % */

/* Thread entry */

void sht30_thread_entry(void *parameter);

/* Init: create and start the SHT30 acquisition thread */

rt_err_t sht30_init(void);

#endif /* APPLICATIONS_APPS_SHT30APP_H_ */

// File: applications/Apps/stressSensorApp.c

/*

* Copyright (c) 2006-2021, RT-Thread Development Team

*

* SPDX-License-Identifier: Apache-2.0

*

* Change Logs:

* Date           Author          Notes

* 2026-09-05     Administrator   initial stress sensor application

* 2026-09-20     Codex           use RDF-TC25/RDD-DH register protocol

*/

#include "stressSensorApp.h"

#include "freeModbusApp.h"

float g_stress_value_n = 0.0f;

int32_t g_stress_raw_value = 0;

uint16_t g_stress_status = 0;

uint16_t g_stress_unit = STRESS_UNIT_NONE;

uint8_t g_stress_decimal_places = 0;

rt_bool_t g_stress_sensor_online = RT_FALSE;

rt_bool_t g_stress_value_valid = RT_FALSE;

static const uint32_t stress_pow10_table[] =

{

1U, 10U, 100U, 1000U, 10000U, 100000U, 1000000U, 10000000U

};

static eMBMasterReqErrCode stress_read_registers(uint8_t slave_addr,

uint16_t *status,

uint16_t *unit,

uint16_t *reg_high,

uint16_t *reg_low)

{

eMBMasterReqErrCode error_code;

error_code = eMBMasterReqReadHoldingRegister(slave_addr,

STRESS_STATUS_REG,

1,

RT_WAITING_FOREVER);

if (error_code != MB_MRE_NO_ERR)

return error_code;

*status = usMRegHoldBuf[slave_addr - 1][STRESS_STATUS_REG];

error_code = eMBMasterReqReadHoldingRegister(slave_addr,

STRESS_UNIT_REG,

1,

RT_WAITING_FOREVER);

if (error_code != MB_MRE_NO_ERR)

return error_code;

*unit = usMRegHoldBuf[slave_addr - 1][STRESS_UNIT_REG];

error_code = eMBMasterReqReadHoldingRegister(slave_addr,

STRESS_VALUE_REG,

STRESS_VALUE_REG_NUM,

RT_WAITING_FOREVER);

if (error_code != MB_MRE_NO_ERR)

return error_code;

*reg_high = usMRegHoldBuf[slave_addr - 1][STRESS_VALUE_REG];

*reg_low = usMRegHoldBuf[slave_addr - 1][STRESS_VALUE_REG + 1];

return MB_MRE_NO_ERR;

}

static rt_bool_t stress_convert_to_n(int32_t raw_value,

uint8_t decimal_places,

uint16_t unit,

float *value_n)

{

float displayed_value;

if (decimal_places >= (sizeof(stress_pow10_table) /

sizeof(stress_pow10_table[0])))

return RT_FALSE;

displayed_value = (float)raw_value /

(float)stress_pow10_table[decimal_places];

switch (unit)

{

case STRESS_UNIT_N:

*value_n = displayed_value;

break;

case STRESS_UNIT_T:

*value_n = displayed_value * 9806.65f;

break;

case STRESS_UNIT_KG:

*value_n = displayed_value * 9.80665f;

break;

case STRESS_UNIT_G:

*value_n = displayed_value * 0.00980665f;

break;

default:

return RT_FALSE;

}

return RT_TRUE;

}

static rt_bool_t stress_decode_measurement(uint16_t status,

uint16_t unit,

uint16_t reg_high,

uint16_t reg_low,

int32_t *raw_value,

uint8_t *decimal_places,

float *value_n)

{

uint32_t combined = ((uint32_t)reg_high << 16) | (uint32_t)reg_low;

*raw_value = (int32_t)combined;

*decimal_places = (uint8_t)(status & STRESS_DECIMAL_MASK);

return stress_convert_to_n(*raw_value, *decimal_places, unit, value_n);

}

rt_err_t stress_sensor_poll(void)

{

eMBMasterReqErrCode error_code;

uint16_t status;

uint16_t unit;

uint16_t reg_high;

uint16_t reg_low;

int32_t raw_value;

uint8_t decimal_places;

float value_n;

error_code = stress_read_registers(STRESS_SLAVE_ADDR,

&status,

&unit,

&reg_high,

&reg_low);

if (error_code != MB_MRE_NO_ERR)

{

g_stress_sensor_online = RT_FALSE;

g_stress_value_valid = RT_FALSE;

return -RT_ERROR;

}

g_stress_sensor_online = RT_TRUE;

g_stress_status = status;

g_stress_unit = unit;

g_stress_value_valid = stress_decode_measurement(status,

unit,

reg_high,

reg_low,

&raw_value,

&decimal_places,

&value_n);

g_stress_raw_value = raw_value;

g_stress_decimal_places = decimal_places;

if (g_stress_value_valid)

g_stress_value_n = value_n;

return RT_EOK;

}

// File: applications/Apps/stressSensorApp.h

/*

* Copyright (c) 2006-2021, RT-Thread Development Team

*

* SPDX-License-Identifier: Apache-2.0

*

* Change Logs:

* Date           Author          Notes

* 2026-09-05     Administrator   initial stress sensor application

* 2026-09-20     Codex           use RDF-TC25/RDD-DH register protocol

*/

#ifndef APPLICATIONS_APPS_STRESSSENSORAPP_H_

#define APPLICATIONS_APPS_STRESSSENSORAPP_H_

#include "heads.h"

/********************* RDF-TC25 / RDD-DH MODBUS ************************/

/*

* Bus parameters: Modbus RTU, 9600 baud, 8 data bits, no parity, 1 stop bit.

* The instrument ships/currently responds as slave 1. Slave 1 is already

* used by the water meter on this bus, so configure the RDD-DH as slave 3

* before connecting both devices to UART3 at the same time.

*/

#define STRESS_SLAVE_ADDR           3

/* Holding registers read with function code 0x03. */

#define STRESS_STATUS_REG           0x0008

#define STRESS_VALUE_REG            0x0050

#define STRESS_VALUE_REG_NUM        2

#define STRESS_UNIT_REG             0x0068

#define STRESS_DECIMAL_MASK         0x0007

#define STRESS_UNIT_NONE            0

#define STRESS_UNIT_G               1

#define STRESS_UNIT_KG              2

#define STRESS_UNIT_T               3

#define STRESS_UNIT_N               4

/* Latest measurement in newtons and protocol metadata. */

extern float g_stress_value_n;

extern int32_t g_stress_raw_value;

extern uint16_t g_stress_status;

extern uint16_t g_stress_unit;

extern uint8_t g_stress_decimal_places;

extern rt_bool_t g_stress_sensor_online;

extern rt_bool_t g_stress_value_valid;

/* Called by the shared UART3 Modbus polling thread. */

rt_err_t stress_sensor_poll(void);

#endif /* APPLICATIONS_APPS_STRESSSENSORAPP_H_ */

// File: applications/Apps/uartApp.c

﻿/*

* Copyright (c) 2006-2021, RT-Thread Development Team

*

* SPDX-License-Identifier: Apache-2.0

*

* Change Logs:

* Date           Author       Notes

* 2025-01-18     Administrator       the first version

*/

#include "uartApp.h"

#define UART4_DEVICE_NAME "uart4" /* use UART4 */

#define UART5_DEVICE_NAME "uart5" /* use UART5 */

/* one-byte temporary receive buffers */

char uart4_buffer;

char uart5_buffer;

/* receive data buffers */

char uart4_recived_data[UART4_RECIVED_DATA_SIZE];

int uart4_recived_data_index = 0;

char uart5_recived_data[UART5_RECIVED_DATA_SIZE];

int uart5_recived_data_index = 0;

/* devices */

rt_device_t uart4_device;

rt_device_t uart5_device;

/* serial configurations */

struct serial_configure uart4_config = RT_SERIAL_CONFIG_DEFAULT;

struct serial_configure uart5_config = RT_SERIAL_CONFIG_DEFAULT;

/* semaphores */

struct rt_semaphore uart4_sem;

struct rt_semaphore uart5_sem;

/* receive indication callbacks */

rt_err_t uart4_rx_ind(rt_device_t dev, rt_size_t size)

{

rt_sem_release(&uart4_sem);

return RT_EOK;

}

rt_err_t uart5_rx_ind(rt_device_t dev, rt_size_t size)

{

rt_sem_release(&uart5_sem);

return RT_EOK;

}

void uart4_init()

{

uart4_device = rt_device_find(UART4_DEVICE_NAME);  /* find UART4 device */

if (uart4_device == RT_NULL)

{

/* rt_kprintf("UART4 device not found!\n"); */

return;

}

uart4_config.baud_rate = 115200;

uart4_config.bufsz = 256;

/* apply configuration */

rt_device_control(uart4_device, RT_DEVICE_CTRL_CONFIG, &uart4_config);

/* open device */

rt_device_open(uart4_device, RT_DEVICE_OFLAG_RDWR | RT_DEVICE_FLAG_INT_RX);

rt_device_set_rx_indicate(uart4_device, uart4_rx_ind);

rt_sem_init(&uart4_sem, "rx_uart4_sem", 0, RT_IPC_FLAG_FIFO);

}

void uart5_init()

{

uart5_device = rt_device_find(UART5_DEVICE_NAME);  /* find UART5 device */

if (uart5_device == RT_NULL)

{

/* rt_kprintf("UART5 device not found!\n"); */

return;

}

uart5_config.baud_rate = 115200;  /* LoRa ATK-LORA-01 baud rate */

uart5_config.bufsz = 256;

/* apply configuration */

rt_device_control(uart5_device, RT_DEVICE_CTRL_CONFIG, &uart5_config);

/* open device */

rt_device_open(uart5_device, RT_DEVICE_OFLAG_RDWR | RT_DEVICE_FLAG_INT_RX);

rt_device_set_rx_indicate(uart5_device, uart5_rx_ind);

rt_sem_init(&uart5_sem, "rx_uart5_sem", 0, RT_IPC_FLAG_FIFO);

}

/* send text through UART4 */

void uart4_send(const char *data)

{

rt_device_write(uart4_device, 0, data, strlen(data));

}

/* send text through UART5 */

void uart5_send(const char *data)

{

rt_device_write(uart5_device, 0, data, strlen(data));

}

/* send binary data through UART5 */

void uart5_send_bytes(const rt_uint8_t *data, rt_size_t len)

{

rt_device_write(uart5_device, 0, data, len);

}

/* UART4 receive thread */

void uart4_thread_entry(void *parameter)

{

uart4_init();

uart4_send("hello uart4!\n");

while(1)

{

/* read one byte from UART device */

while (rt_device_read(uart4_device, 0, &uart4_buffer, 1) != 1)

{

rt_sem_take(&uart4_sem, RT_WAITING_FOREVER);

}

/* store received byte into buffer */

if (uart4_recived_data_index < UART4_RECIVED_DATA_SIZE - 1)

{

uart4_recived_data[uart4_recived_data_index++] = uart4_buffer;

}

}

}

/* UART5 receive thread */

void uart5_thread_entry(void *parameter)

{

uart5_init();

uart5_send("hello uart5!\n");

while(1)

{

/* read one byte from UART device */

while (rt_device_read(uart5_device, 0, &uart5_buffer, 1) != 1)

{

rt_sem_take(&uart5_sem, RT_WAITING_FOREVER);

}

/* store received byte into buffer */

if (uart5_recived_data_index < UART5_RECIVED_DATA_SIZE - 1)

{

uart5_recived_data[uart5_recived_data_index++] = uart5_buffer;

}

/* rt_kprintf("uart5_recived_data:%s\n", uart5_recived_data); */

}

}

void uart4_buffer_clear()

{

/* clear buffer before next receive operation */

memset(uart4_recived_data, 0x00, sizeof(uart4_recived_data));

uart4_recived_data_index = 0;

}

void uart5_buffer_clear()

{

/* clear buffer before next receive operation */

memset(uart5_recived_data, 0x00, sizeof(uart5_recived_data));

uart5_recived_data_index = 0;

}

// File: applications/Apps/uartApp.h

/*

* Copyright (c) 2006-2021, RT-Thread Development Team

*

* SPDX-License-Identifier: Apache-2.0

*

* Change Logs:

* Date           Author       Notes

* 2025-01-18     Administrator       the first version

*/

#ifndef APPLICATIONS_APPS_UARTAPP_H_

#define APPLICATIONS_APPS_UARTAPP_H_

#include "heads.h"

#define UART4_RECIVED_DATA_SIZE 256    /* UART4 receive buffer size */

#define UART5_RECIVED_DATA_SIZE 128    /* UART5 receive buffer size */

extern char uart4_recived_data[UART4_RECIVED_DATA_SIZE];

extern int uart4_recived_data_index;

extern char uart5_recived_data[UART5_RECIVED_DATA_SIZE];

extern int uart5_recived_data_index;

/* callbacks */

rt_err_t uart4_rx_ind(rt_device_t dev, rt_size_t size);

rt_err_t uart5_rx_ind(rt_device_t dev, rt_size_t size);

/* init */

void uart4_init();

void uart5_init();

/* send */

void uart4_send(const char *data);

void uart5_send(const char *data);

void uart5_send_bytes(const rt_uint8_t *data, rt_size_t len);

/* threads */

void uart4_thread_entry(void *parameter);

void uart5_thread_entry(void *parameter);

/* buffer helpers */

void uart4_buffer_clear();

void uart5_buffer_clear();

#endif /* APPLICATIONS_APPS_UARTAPP_H_ */

// File: applications/Apps/wifiApp.c

﻿/*

* Copyright (c) 2006-2021, RT-Thread Development Team

*

* SPDX-License-Identifier: Apache-2.0

*

* Change Logs:

* Date           Author       Notes

* 2025-02-24     Administrator       the first version

*/

#include "wifiApp.h"

//上云标志

char task_flag = 0;

//上报消息json

char MQTT_json[128];

void if_is_ok()

{

if (strstr((const char*)uart4_recived_data,"OK") != NULL) {

/* rt_kprintf("字符串匹配：OK\n"); */

task_flag++;

} else if(strstr((const char*)uart4_recived_data,"ERROR") != NULL) {

/* rt_kprintf("出错，重试...\n"); */

/* rt_kprintf("字符串：%s\n", uart4_recived_data); */

task_flag = 0;

} else {

/* rt_kprintf("字符串不匹配\n"); */

/* rt_kprintf("字符串：%s\n", uart4_recived_data); */

}

uart4_buffer_clear();

/* rt_kprintf("task_flag：%d\n", task_flag); */

}

// 创建一个线程来执行上云操作

void wifi_thread_entry(void *parameter)

{

char command[512] = {0};

while(task_flag == 0)

{

while(task_flag == 0)       //0.重置8266

{

// 重启设备

uart4_send("AT+RST\r\n");

// 等待一定时间让串口接收数据

rt_thread_mdelay(500);

if_is_ok();

}

while(task_flag == 1)       //1.设置station模式

{

// 设置为 Station 模式

uart4_send("AT+CWMODE=1\r\n");

// 等待一定时间让串口接收数据

rt_thread_mdelay(500);

if_is_ok();

}

while(task_flag == 2)       //2.连接wifi

{

// 连接WiFi

memset(command, 0, sizeof(command));

rt_sprintf(command, "AT+CWJAP=\"%s\",\"%s\"\r\n", WIFI_SSID, WIFI_PASSWORD);

/* rt_kprintf("command:%s\n", command); */

uart4_send(command);

// 等待一定时间让串口接收数据

rt_thread_mdelay(1000);

if_is_ok();

}

while(task_flag == 3)       //3.设置三元组用户名和密码

{

memset(command, 0, sizeof(command));

rt_sprintf(command, "AT+MQTTUSERCFG=0,1,\"NULL\",\"%s\",\"%s\",0,0,\"\"\r\n",

MQTT_USERNAME, MQTT_PASSWORD);

uart4_send(command);

/* rt_kprintf("command:%s\n", command); */

// 等待一定时间让串口接收数据

rt_thread_mdelay(500);

if_is_ok();

}

while(task_flag == 4)       //4.设置ClientID

{

memset(command, 0, sizeof(command));

rt_sprintf(command, "AT+MQTTCLIENTID=0,\"%s\"\r\n", MQTT_CLIENT_ID);

uart4_send(command);

/* rt_kprintf("command:%s\n", command); */

// 等待一定时间让串口接收数据

rt_thread_mdelay(1500);

if_is_ok();

}

while(task_flag == 5)       //5.绑定MQTT接入地址

{

memset(command, 0, sizeof(command));

rt_sprintf(command, "AT+MQTTCONN=0,\"%s\",1883,1\r\n", MQTT_IP);

uart4_send(command);

// 等待一定时间让串口接收数据

rt_thread_mdelay(500);

if_is_ok();

}

while(task_flag == 6)       //6.订阅主题

{

memset(command, 0, sizeof(command));

rt_sprintf(command, "AT+MQTTSUB=0,\"oc/devices/%s/sys/properties/report\",1\r\

n", MQTT_USERNAME);

uart4_send(command);

// 等待一定时间让串口接收数据

rt_thread_mdelay(500);

if_is_ok();

}

while(task_flag == 7)       //7.消息上报

{

memset(command, 0, sizeof(command));

snprintf(command, sizeof(command), "AT+MQTTPUB=0,\"$oc/devices/%s/sys/properti

es/report\",\"{\\\"services\\\":[{\\\"service_id\\\":\\\"Node_1\\\"\\,\\\"properties\\\":{

\\\"Flow\\\":%.2f\\,\\\"Flame\\\":%d\\,\\\"Methane\\\":%.2f}}]}\",0,0\r\n", MQTT_USERNAME,

node[0].Flow, node[0].Flame, node[0].Methane);

uart4_send(command);

rt_thread_mdelay(100);

memset(command, 0, sizeof(command));

snprintf(command, sizeof(command), "AT+MQTTPUB=0,\"$oc/devices/%s/sys/properti

es/report\",\"{\\\"services\\\":[{\\\"service_id\\\":\\\"Node_1\\\"\\,\\\"properties\\\":{

\\\"current_A\\\":%.2f\\,\\\"current_B\\\":%.2f\\,\\\"current_C\\\":%.2f}}]}\",0,0\r\n", M

QTT_USERNAME, node[0].CH1_A[0], node[0].CH1_A[1], node[0].CH1_A[2]);

uart4_send(command);

rt_thread_mdelay(100);

//            memset(command, 0, sizeof(command));

//            snprintf(command, sizeof(command), "AT+MQTTPUB=0,\"$oc/devices/%s/sys/proper

ties/report\",\"{\\\"services\\\":[{\\\"service_id\\\":\\\"Node_2\\\"\\,\\\"properties\\\"

:{\\\"Flow\\\":%.2f\\,\\\"Flame\\\":%d\\,\\\"Methane\\\":%.2f}}]}\",0,0\r\n", MQTT_USERNAM

E, node[1].Flow, node[1].Flame, node[1].Methane);

//            uart4_send(command);

//            rt_thread_mdelay(100);

//            memset(command, 0, sizeof(command));

//            snprintf(command, sizeof(command), "AT+MQTTPUB=0,\"$oc/devices/%s/sys/proper

ties/report\",\"{\\\"services\\\":[{\\\"service_id\\\":\\\"Node_2\\\"\\,\\\"properties\\\"

:{\\\"current_A\\\":%.2f\\,\\\"current_B\\\":%.2f\\,\\\"current_C\\\":%.2f}}]}\",0,0\r\n",

MQTT_USERNAME, node[1].CH1_A[0], node[1].CH1_A[1], node[1].CH1_A[2]);

//            uart4_send(command);

// 等待一定时间让串口接收数据

rt_thread_mdelay(500);

/* rt_kprintf("upload data\n"); */

//            if_is_ok();

}

}

}

// File: applications/Apps/wifiApp.h

/*

* Copyright (c) 2006-2021, RT-Thread Development Team

*

* SPDX-License-Identifier: Apache-2.0

*

* Change Logs:

* Date           Author       Notes

* 2025-02-24     Administrator       the first version

*/

#ifndef APPLICATIONS_APPS_WIFIAPP_H_

#define APPLICATIONS_APPS_WIFI_H_

#include "heads.h"

/*************************************************

* 结尾有\r\n

1. AT+CWMODE=1

2. AT+CWJAP="cdut-yb","cdutyb218"

3. AT+MQTTUSERCFG=0,1,"NULL","67bedcb724d772325520a4c7_pipe_1","43846f59b20582164b1e50874a

a3ef815af9d01c9cc35a0dc74232f72f163683",0,0,""

4. AT+MQTTCLIENTID=0,"67bedcb724d772325520a4c7_pipe_1_0_0_2025022804"

5. AT+MQTTCONN=0,"48931f72a4.st1.iotda-device.cn-north-4.myhuaweicloud.com",1883,1

6. AT+MQTTSUB=0,"oc/devices/67bedcb724d772325520a4c7_pipe_1/sys/properties/report",1

7. AT+MQTTPUB=0,"$oc/devices/67bedcb724d772325520a4c7_pipe_1/sys/properties/report","{\"se

rvices\":[{\"service_id\":\"Node_1\"\,\"properties\":{\"Flow\":12.23\,\"Flame\": 1\,\"Meth

ane\":3.23\,\"current_A\":2\,\"current_B\":3.2\,\"current_C\":3.5}}]}",0,0

*************************************************/

#define WIFI_SSID    "Win10-2018PYBHT 2565"   // Wi-Fi 名称    "cdut-yb"    "Win10-2018PYB

HT 2565"

#define WIFI_PASSWORD "1&H09h78" // Wi-Fi 密码   "cdutyb218"   "1&H09h78"

#define MQTT_USERNAME "67bedcb724d772325520a4c7_pipe_1" // 三元组用户名

#define MQTT_PASSWORD "43846f59b20582164b1e50874aa3ef815af9d01c9cc35a0dc74232f72f163683"//

密码

#define MQTT_CLIENT_ID "67bedcb724d772325520a4c7_pipe_1_0_0_2025022804"//ClientID

#define MQTT_IP "48931f72a4.st1.iotda-device.cn-north-4.myhuaweicloud.com" // MQTT地址

//#define MQTT_SUB_TOPIC "oc/devices/67bedcb724d772325520a4c7_pipe_1/sys/properties/report

" // MQTT订阅topic

void if_is_ok();

void wifi_thread_entry(void *parameter);

#endif /* APPLICATIONS_APPS_WIFIAPP_H_ */

// File: applications/heads.h

/*

* Copyright (c) 2006-2021, RT-Thread Development Team

*

* SPDX-License-Identifier: Apache-2.0

*

* Change Logs:

* Date           Author       Notes

* 2025-01-16     Administrator       the first version

*/

#ifndef APPLICATIONS_HEADS_H_

#define APPLICATIONS_HEADS_H_

#include <rtthread.h>

#include <rtdevice.h>

#include "mb.h"

#include "mb_m.h"

#include "mbconfig.h"

#include "user_mb_app.h"

#include "freeModbusApp.h"

#include "rtdevice.h"

#include <stdlib.h>

#include <stdio.h>

#include <string.h>

#include "uartApp.h"

#include "wifiApp.h"

#include "MethaneSensorApp.h"

#include "NodeApp.h"

#include "linesensor.h"

#include "huaweiCloudApp.h"

#include "loraApp.h"

#include "sht30App.h"

#include "o2SensorApp.h"

#include "ethApp.h"

#include "displacementSensorApp.h"

#include "stressSensorApp.h"

#endif /* APPLICATIONS_HEADS_H_ */

// File: applications/main.c

﻿/*

* Copyright (c) 2006-2025, RT-Thread Development Team

*

* SPDX-License-Identifier: Apache-2.0

*

* Change Logs:

* Date           Author       Notes

* 2025-01-13     RT-Thread    first version

#include <rtdbg.h>

* 2026-02-04     ideapad15s   add Huawei Cloud IoT

#define DBG_TAG "main"

* 2026-04-13     ideapad15s   add LoRa ATK-LORA-01

*/

#include "MethaneSensorApp.h"

#include "linesensor.h"

#include "huaweiCloudApp.h"

#include "loraApp.h"

#include "sht30App.h"

#include "o2SensorApp.h"

#include "ethApp.h"

#include "displacementSensorApp.h"

#include "stressSensorApp.h"

#define DBG_LVL DBG_LOG

static void uart2_receive_thread_entry(void *parameter)

{

/* call receive function in infinite wait mode */

uart2_receive_and_print(-1);

}

/* Gas sensor: 3.3V power, USART2 */

/* Five-channel flame sensor: analog outputs A1-A5, ADC */

/* MODBUS: fixed data frame (CRC cannot be changed), USART3 as signal bus */

int main(void)

{

/* ============================================================

* UART assignment:

* UART2 (PD5/PD6)    - Gas sensor (19200 baud)

* UART3 (PB10/PB11)  - MODBUS RTU master (9600 baud, water/displacement/stress/ammete

r)

* UART4 (PA12/PA11)  - ESP8266 WiFi module (115200 baud)

* UART5 (PC12/PD2)   - ATK-LORA-01 LoRa module (115200 baud)

* ============================================================ */

/* UART4 receive thread - for ESP8266 communication */

rt_thread_t uart4_thread = rt_thread_create("uart4_rx", uart4_thread_entry, RT_NULL, 1

024, 25, 10);

if (uart4_thread != RT_NULL)

rt_thread_startup(uart4_thread);

/* Huawei Cloud thread - report sensor data */

huawei_cloud_init();

/* MODBUS poll thread (UART3) */

rt_thread_t tid1 = rt_thread_create("md_m_poll", mb_master_poll, RT_NULL, 512, MB_POLL

_THREAD_PRIORITY, 10);

if (tid1 != RT_NULL)

rt_thread_startup(tid1);

/* MODBUS master send thread (UART3) */

rt_thread_t tid2 = rt_thread_create("md_m_send", send_thread_entry, RT_NULL, 1024, MB_

SEND_THREAD_PRIORITY - 2, 10);

if (tid2 != RT_NULL)

rt_thread_startup(tid2);

/* Five-channel flame sensor ADC init and read thread (A1-A5) */

line_sensor_init();

/* SHT30 temperature/humidity sensor (I2C1: PB6 SCL, PB7 SDA) */

if (sht30_init() != RT_EOK)

rt_kprintf("[main] E: SHT30 init failed!\n");

else

rt_kprintf("[main] I: SHT30 initialized on i2c1\n");

/* O2 sensor (ADC1 channel 18, PA4) */

if (o2_sensor_init() != RT_EOK)

rt_kprintf("[main] E: O2 sensor init failed!\n");

else

rt_kprintf("[main] I: O2 sensor initialized on adc1 ch18\n");

/* Ethernet TCP client - fiber optic main link */

if (eth_app_init() != RT_EOK)

rt_kprintf("[ETH] Failed to initialize Ethernet TCP client!\n");

else

rt_kprintf("[ETH] Ethernet TCP client initialized.\n");

/* LoRa send thread (UART5) - ATK-LORA-01 transparent mode */

rt_thread_t lora_thread = rt_thread_create("lora_tx", lora_thread_entry, RT_NULL, 1024

, 25, 10);

if (lora_thread != RT_NULL)

rt_thread_startup(lora_thread);

/* UART2 receive thread - gas sensor (3.3V power) */

rt_thread_t uart2_thread = rt_thread_create("uart2_rx",uart2_receive_thread_entry,RT_N

ULL,1024,25,10);

if (uart2_thread != RT_NULL)

rt_thread_startup(uart2_thread);

/* ============================================================

* Displacement sensor test thread (CN2 interface, RS485)

* 测试完成，已集成到 freeModbusApp.c 的主轮询中

* ============================================================ */

// rt_thread_t disp_test_thread = rt_thread_create("disp_test",

//                                                  displacement_sensor_test_thread_en

try,

//                                                  RT_NULL,

//                                                  1024,

//                                                  26,

//                                                  10);

// if (disp_test_thread != RT_NULL)

// {

//     rt_thread_startup(disp_test_thread);

//     rt_kprintf("[DISP] Displacement sensor test thread started.\n");

// }

// else

// {

//     rt_kprintf("[DISP] Failed to create displacement sensor test thread!\n");

// }

return RT_EOK;

}